CEN

CWA 16374

WORKSHOP	July 2023

AGREEMENT

ICS 35.240.50

English version

Extensions for Financial Services (XFS) interface specification

Release 3.20

Clarifications for Device Class Interfaces

Revision 3.20.24

July 2023

This CEN Workshop Agreement has been drafted and approved by a Workshop of representatives of interested parties, the constitution of which is indicated in the foreword of this Workshop Agreement.

The formal process followed by the Workshop in the development of this Workshop Agreement has been endorsed by the National Members of CEN but neither the National Members of CEN nor the CEN Management Centre can be held accountable for the technical content of this CEN Workshop Agreement or possible conflicts with standards or legislation.

This CEN Workshop Agreement can in no way be held as being an official standard developed by CEN and its Members.

This CEN Workshop Agreement is publicly available as a reference document from the CEN Members National Standard Bodies.

CEN members are the national standards bodies of Austria, Belgium, Cyprus, Czech Republic, Denmark, Estonia, Finland, France, Germany, Greece, Hungary, Iceland, Ireland, Italy, Latvia, Lithuania, Luxembourg, Malta, Netherlands, Norway, Poland, Portugal, Slovakia, Slovenia, Spain, Sweden, Switzerland and United Kingdom.

EUROPEAN COMMITTEE FOR STANDARDIZATION

COMITÉ EUROPÉEN DE NORMALISATION

EUROPÄISCHES KOMITEE FÜR NORMUNG

Management Centre: rue de Stassart, 36 B-1050 Brussels

© 2023 CEN All rights of exploitation in any form and by any means reserved worldwide for CEN national Members.

Ref. No. CWA 16374:2011 E

Table of Contents

Introduction	5

Generic Clarifications	6

Device Classes	7

Application Programming Interface / Service Provider Interface	7

Clarifications for Synchronous Functions	7

Clarifications for Configuration Information	7

Clarifications for Application Processes, Threads and Blocking Functions	7

Clarifications for WFS_SYSE_HARDWARE_ERROR, WFS_SYSE_SOFTWARE_ERROR, WFS_SYSE_USER_ERROR and WFS_SYSE_FRAUD_ATTEMPT	10

Clarifications for WFS_SYSE_DEVICE_STATUS	11

Clarifications for WFSOpen	12

Clarifications for WFSAsyncOpen	12

Clarifications for WFPOpen	12

Clarifications for WFMOutputTraceData	12

Clarifications for WFMCreateKey	12

Clarifications for WFMDeleteKey	13

Clarifications for WFMDeleteValue	13

Clarifications for WFMOpenKey	13

Clarifications for WFMQueryValue	13

Clarifications for WFMSetValue	13

Clarifications for XFSAPI.H	13

Clarifications for XFSADMIN.H	14

Clarifications for XFSCONF.H	14

Clarifications for XFSSPI.H	14

Printers and Scanners	15

Clarifications for WFS_INF_PTR_STATUS	15

Clarifications for WFS_INF_PTR_CAPABILITIES	15

Clarifications for WFS_INF_PTR_QUERY_FIELD	15

Clarifications for WFS_CMD_PTR_READ_FORM	15

Clarifications for WFS_SRVE_PTR_MEDIATAKEN	16

Clarifications for WFS_SRVE_PTR_MEDIAAUTORETRACTED	16

Clarifications for Definition Syntax	16

Clarifications for Form Definition	17

Clarifications for Subform Definition	17

Clarifications for Field Definition	17

Identification Card Units	18

Clarifications for WFS_CMD_IDC_RETAIN_CARD	18

Clarifications for WFS_CMD_IDC_CHIP_POWER	18

Clarifications for C - Header File	18

Cash Dispensers	19

Clarifications for WFS_INF_CDM_STATUS	19

Clarifications for WFS_INF_CDM_CAPABILITIES	19

Clarifications for WFS_INF_CDM_CASH_UNIT_INFO	19

Clarifications for WFS_INF_CDM_PRESENT_STATUS	20

Clarifications for WFS_CMD_CDM_DISPENSE	21

Clarifications for WFS_CMD_CDM_PRESENT	21

Clarifications for WFS_CMD_CDM_RETRACT	21

Clarifications for WFS_CMD_CDM_END_EXCHANGE	21

Clarifications for WFS_CMD_CDM_CALIBRATE_CASH_UNIT	22

Clarifications for WFS_CMD_CDM_PREPARE_DISPENSE	22

Clarifications for WFS_CMD_CDM_RESET	22

Clarifications for WFS_EXEE_CDM_INCOMPLETEDISPENSE	23

Clarifications for C - Header File	23

Personal Identification Number Keypads (PIN Pads)	24

Clarifications for Section 3. References	24

Clarifications for WFS_INF_PIN_CAPABILITIES	24

Clarifications for WFS_CMD_PIN_CRYPT	24

Clarifications for WFS_CMD_PIN_GET_DATA	24

Clarifications for WFS_CMD_PIN_GET_PINBLOCK	25

Clarifications for WFS_CMD_CDM_RESET	25

Clarifications for WFS_CMD_PIN_IMPORT_RSA_ENCIPHERED_PKCS7_KEY_EX	25

Clarifications for C - Header File	25

Clarifications for Remote Key Exchange	27

Clarifications for Replace Certificate	28

Clarifications for Luxemburg Protocol	28

Clarifications for Luxemburg-specific Header File	28

Check Reader/Scanner	29

Depository Unit	30

Text Terminal Unit	31

Sensors and Indicators Units	32

Clarifications for Enhanced Audio Controller Overview	32

Clarifications for WFS_INF_SIU_STATUS	32

Clarifications for C - Header file	32

Vendor Dependent Mode	34

Clarifications for WFS_CMD_VDM_ENTER_MODE_ACK	34

Clarifications for WFS_SRVE_VDM_INTERFACE_ CHANGED	34

Cameras	35

Clarifications for WFS_INF_CAM_STATUS	35

Alarms	36

Card Embossing Unit	37

Cash In Module	38

Clarifications for WFS_INF_CIM_STATUS	38

Clarifications for WFS_INF_CIM_CAPABILITIES	38

Clarifications for WFS_INF_CIM_CASH_UNIT_INFO	39

Clarifications for WFS_INF_CIM_BANKNOTE_TYPES	40

Clarifications for WFS_INF_CIM_CASH_IN_STATUS	40

Clarifications for WFS_INF_CIM_GET_P6_SIGNATURE	41

Clarifications for WFS_INF_CIM_ GET_ITEMS_INFO	41

Clarifications for WFS_CMD_CIM_CASH_IN_START	41

Clarifications for WFS_CMD_CIM_CASH_IN	42

Clarifications for WFS_CMD_CIM_OPEN_SHUTTER	42

Clarifications for WFS_CMD_CIM_RESET	42

Clarifications for WFS_CMD_CIM_CONFIGURE_NOTETYPES	43

Clarifications for WFS_CMD_CIM_REPLENISH	43

Clarifications for WFS_CMD_CIM_SET_CASH_IN_LIMIT	43

Clarifications for WFS_CMD_CIM_PRESENT_MEDIA	44

Clarifications for WFS_SRVE_CIM_ITEMSTAKEN	44

Clarifications for WFS_EXEE_CIM_INPUTREFUSE	44

Clarifications for ATM Cash-In Transaction Flow - Application Guidelines	45

8.16 Multiple Bunch Timeout Handling	45

No Items Inserted	45

First Bunch Not Taken	46

Last Bunch Taken	47

Clarifications for Mixed Media Cancellation by Customer	48

Clarifications for Mixed Media Cancellation by Customer on Cash Part Only	48

Clarifications for Mixed Media Multiple Refused Items	48

Card Dispenser	50

Bar Code Reader	51

Clarifications for C - Header File	51

Item Processing Module	52

Clarifications for WFS_INF_IPM_STATUS	52

Clarifications for WFS_INF_IPM_TRANSACTION_STATUS	52

Clarifications for WFS_INF_IPM_MEDIA_IN	54

Clarifications for WFS_CMD_IPM_READ_IMAGE	54

Clarifications for WFS_SRVE_IPM_MEDIATAKEN	54

Clarifications for WFS_EXEE_IPM_MEDIAREFUSED	54

Clarifications for WFS_EXEE_IPM_MEDIADATA	55

# Introduction

These release notes provide clarifications and explanations for the Device Class Interface Programmer’s References Revision 3.20. Rather than updating the Device Class Interface specifications each time a new clarification is required, CEN/ISSS established release notes should be developed that aggregates the clarifications and explanations. These release notes serve that purpose. The release notes will provide clarifications of problems reported to CEN/ISSS which do not require functional changes. When a Device Class Interface Programmer’s Reference is updated for functional changes then all clarifications contained in these release notes at the current revision level for that Device Class will be incorporated into the new revision.

The clarification will be incorporated into the appropriate section copied from the affected Device Class Interface Programmer’s Reference and be printed as bold and underlined.

# Generic Clarifications

No clarifications available.

# Device Classes

## Application Programming Interface / Service Provider Interface

Class Name		API/SPI

### Clarifications for Synchronous Functions

…

If a blocking operation is not completed immediately in a Windows 3.x system, the XFS Manager executes a Windows message loop on behalf of the calling thread, thereby keeping the Windows system running. See Section  for a more detailed discussion of process, threads and message loops. In Windows NT, tThe calling application thread is blocked on request completion. A thread may have only one blocking XFS call outstanding at any one time. See Section  for additional discussion of the management of synchronous functions, including replacement of the default message loop.

### Clarifications for Configuration Information

…

These functions are used by Service Providers and applications to write and retrieve the configuration information for an XFS subsystem, which is stored in a hierarchical structure called the Windows Registry. The structure and the functions are based on the Win32/Win64 Registry architecture and API functions, and are implemented in Windows NT/98 and future versions of Windows using the Registry and the associated functions.

…

The local PC dependent configuration information is stored beneath the following Registry key. A pre-defined handle (WFS_CFG_HKEY_MACHINE_XFS_ROOT) can be used to access this key in the configuration functions defined in Section 8.

User dependent configuration information is stored in the HKEY_USERS section of the Registry. Pre-defined handles (WFS_CFG_HKEY_USER_DEFAULT_XFS_ROOT and WFS_CFG_CURRENT_USER_XFS_ROOT ) can be used to access these keys in the configuration functions defined in Section 8.

### Clarifications for Application Processes, Threads and Blocking Functions

An application process contains one or more threads of execution. The XFS interface is designed to work in both the single-threaded versions of the Windows operating systems (Windows 3.1 and Windows for Workgroups) and in the multi-threaded versions of Windows (Windows NT and future versions of Windows). All references to threads in this document refer to actual threads in multi-threaded Windows environments. In single-threaded environments, “thread” is synonymous with “process.”

Within the XFS Manager, a blocking (synchronous) function is handled as follows:

The XFS Manager initiates the operation, and then enters a loop in which it dispatches any Windows messages (thus yielding the processor to other applications as necessary) and checks for the completion of the operation. When the operation completes, or WFSCancelBlockingCall is invoked, the blocking operation completes with an appropriate result.

- The XFS Manager creates a transitory HWND on the calling thread to receive the completion message for the operation e.g. WFS_EXECUTE_COMPLETE.

- The XFS Manager calls the Service Provider WFP API, passing the transitory HWND.

- The XFS Manager waits for the completion message to be received. It does this by entering a loop equivalent to the following pseudo code, calling the current blocking hook (a Windows message dispatch routine) waiting for the completion message to be received from the Service Provider.

for(;;) {
/* flush messages for good user response */
for(;;) {
BlockingHook();
/* check for WFSCancelBlockingCall() */
if ( operation_cancelled() )
    break;
/* check to see if operation completed */
if( operation_completed() )
    break;    /* normal completion */
}

where the Default Blocking Hook is equivalent to:

BOOL DefaultBlockingHook(void) {
    MSG msg = {0};
    BOOL ret = GetMessage(&msg, NULL, 0, 0);
    if( (int) ret != -1 ) {
        TranslateMessage(&msg);
        DispatchMessage(&msg);
    }
    /* FALSE if we got a WM_QUIT message */
    return( ret );
}

- On reception of the completion message, the XFS Manager exits the loop.

- The XFS Manager destroys the transitory HWND.

- The blocking operation completes. The blocking function return code is copied from the completion message lpWFSResult hResult field. If applicable, the lpWFSResult is also returned.

The thread, on which the blocking function has been called, is not permitted to issue any XFS calls other than the following two specific functions provided to assist the developer in this situation.

- WFSIsBlocking determines whether or not a blocking function is in progress.

- WFSCancelBlockingCall cancels a blocking function in progress.

Any other XFS function, called from a thread with a blocking function in progress, will fail with the error WFS_ERR_OP_IN_PROGRESS.

Developers must be aware that WFSIsBlocking cannot simply be called in a loop waiting for the blocking function to complete. The application must allow the message handler to return to allow control to return to the blocking hook. Otherwise, the blocking function will not complete.

When a Windows message is received for a thread for which a blocking operation is in progress, the thread is not permitted to issue any XFS calls during the processing of the message, other than the two specific functions provided to assist the programmer in this situation:

- WFSIsBlocking determines whether or not a blocking call is in progress.

- WFSCancelBlockingCall cancels a blocking call in progress.

Any other XFS function called when a blocking call is in progress fails with the error WFS_ERR_OP_IN_PROGRESS. This restriction applies to requests for both blocking and non-blocking operations.

Although this mechanism is sufficient for simple applications, it cannot support those applications which require more complex message processing while blocked for a synchronous calla blocking function is executing, such as processing messages relating to MDI (multiple document interface) events, accelerator key translations, and modeless dialogs. For such applications, the XFS API includes the function WFSSetBlockingHook, which allows the programmerdeveloper to define a special routine custom blocking hook which will be called instead of the default message dispatch routine blocking hook described above. This function gives an application the ability to execute its own routine at blocking time in place of the default routine. It is not intended as a mechanism for performing general application functions while blocked; it is still true that the only XFS functions that may be called from a blocking routine are WFSIsBlocking and WFSCancelBlockingCall. The asynchronous versions of the XFS functions must be used to allow an application to continue processing while an operation is in progress. Developers must be aware of their responsibility when replacing the default blocking hook. The developer must ensure:

- All messages are processed in the order received. If not, the potential exists for the Service Provider to be blamed for sending messages in the wrong order e.g. a WFS_EXECUTE_EVENT message after a WFS_EXECUTE_COMPLETE.

- All messages are processed. If not, the potential exists that the thread message queue will fill preventing other messages being added to the queue, including the Service Provider attempt to post the completion message being waited on.

The developer must be aware that replacing the default blocking hook impacts the process. The custom blocking hook will be called from every thread which makes use of XFS blocking functions.

This mechanism is provided to allow a Windows 3.x or Windows for Workgroups application to make blocking calls without blocking the rest of the system. Under Windows NT and future multi-threaded, preemptive versions of Windows, the default blocking action is to suspend the calling application's thread until the request completes. This is because the system is not blocked by a single application waiting for an operation to complete, and hence not calling PeekMessage or GetMessage, which are required in the non-preemptive systems in order to cause the application to yield control.

Therefore, if a single-threaded application is targeted at both single- and multi-threaded environments, and relies on this functionality, it should install a specific blocking hook by calling WFSSetBlockingHook, even if the default hook would suffice. This maximizes the portability of applications that depend on the blocking hook behavior. Programmers who are constrained to use blocking mode - for example, as part of an existing application which is being ported - should be aware of the semantics of blocking operations.

In the XFS implementation in a single-threaded environment, the blocking function operates as follows. When an application requests a blocking XFS API function, the XFS Manager initiates the requested function and then enters a loop which is equivalent to the following pseudo-code:

for(;;) {

/* flush messages for good user response */

DefaultBlockingHook();

/* check for WFSCancelBlockingCall() */

if( operation_cancelled() )

break;

/* check to see if operation completed */

if( operation_complete() )

break;			/* normal completion */

}

The DefaultBlockingHook routine is equivalent to:

BOOL DefaultBlockingHook(void) {

MSG msg;

BOOL ret;

/* Wait for the next message */

ret = GetMessage(&msg, NULL, 0, 0);

if( (int) ret != -1 ) {

TranslateMessage(&msg);

DispatchMessage(&msg);

}

/* FALSE if we got a WM_QUIT message */

return( ret );

}

In a multi-threaded environment, the developer of a multi-threaded application must be aware that it is the responsibility of the application, not the XFS Manager, to synchronize access to a service by multiple threads. Failure to synchronize calls to a service leads to unpredictable results; for example, if two threads "simultaneously" issue WFSExecute requests to send data to the same service, there is no guarantee as to the order in which the data is sent. This is true in general; the application is responsible for coordinating access by multiple threads to any object (e.g. other forms of I/O, such as file I/O), using appropriate synchronization mechanisms. The XFS Manager can not, and will not, address these issues. The possible consequences of failing to observe these rules are beyond the scope of this specification.

In order to allow maximum flexibility in the design and implementation of applications, especially in multi-threaded environments, the concept of "application identity" can optionally be managed explicitly by the application developer using the concept of application handles. See Sections Error: Reference source not found and Error: Reference source not found for additional discussion of this concept.

### Clarifications for WFS_SYSE_HARDWARE_ERROR, WFS_SYSE_SOFTWARE_ERROR, WFS_SYSE_USER_ERROR and WFS_SYSE_FRAUD_ATTEMPT

Field	Description

lpszLogicalName	Pointer to the logical service name of the service that generated the error

lpszPhysicalName	Pointer to the physical service name of the service that generated the error

lpszWorkstationName	Pointer to the name of the workstation in which the logical service name is defined (if any)

lpszAppID	Pointer to the application ID associated with the session that generated the error (if any)

dwAction	The action required to manage the error. Possible values are:

Value	Meaning

WFS_ERR_ACT_NOACTION	No action required or error was autorecovered.

WFS_ERR_ACT_RESET	Reset device to attempt recovery using WFS_CMD_XXX_RESET, but should not be used excessively. Intervention is not required although if repeated attempts are unsuccessful then WFS_ERR_ACT_HWMAINT may be reported.

WFS_ERR_ACT_SWERROR	A software error occurred. Contact software vendor.

WFS_ERR_ACT_CONFIG	A configuration error occurred. Check configuration.

WFS_ERR_ACT_HWCLEAR	Recovery is not possible. A manual intervention for clearing the device is required. This value is only used for hardware errors. This value is typically returned when a hardware error has occurred which requires banking personnel specific maintenance, e.g. ‘replace paper’, or ‘remove cards from retain bin’.

WFS_ERR_ACT_HWMAINT	Recovery is not possible. A technical maintenance intervention is required. This value is only used for hardware errors and fraud attempts. This value is typically returned when a hardware error or fraud attempt has occurred which requires field engineer specific maintenance activity. WFS_CMD_XXX_RESET may be used to attempt recovery after intervention, but should not be used excessively – Vendor Dependent Mode may be required to recover the device.

WFS_ERR_ACT_SUSPEND	Device will attempt auto recovery and will advise any further action required via a Device Status Event.

dwSize	The size in bytes of the following description

lpbDescription	Pointer to a vendor-specific description of the error.

Note:

The following table describes what dwAction may be returned for the various Hardware, Software, User Error and Fraud Attempt Events. The dwAction definitions above give guidance on what an application should do next when one of these events is received. Care should be taken to avoid calling WFS_CMD_XXX_RESET excessively without intervention, as this may lead to damage to the device or media contained in the device if for example media is jammed in the device:

|  | Generated on Hardware Event? | Generated on Software Event? | Generated on User Event? | Generated on Fraud Event? |
| --- | --- | --- | --- | --- |
| _NOACTION | Yes | Yes | Yes | Yes |
| _RESET | Yes | Yes | Yes | No |
| _SWERROR | No | Yes | No | No |
| _CONFIG | Yes | Yes | No | No |
| _HWCLEAR | Yes | No | No | No |
| _HWMAINT | Yes | No | No | Yes |
| _SUSPEND | No | No | Yes | No |

### Clarifications for WFS_SYSE_DEVICE_STATUS

Field	Description

RequestID	(not used)

hService	Service handle identifying the session that created the result, i.e. the service handle of the session that the event is sent to.

tsTimestamp	Time the status change occurred (local time, in a Win32 SYSTEMTIME structure).

hResult	(not used)

u.dwEventID	= WFS_SYSE_DEVICE_STATUS

lpBuffer	Pointer to a WFSDEVSTATUS structure:

typedef struct	wfs_devstatus {

LPSTR			lpszPhysicalName;

LPSTR			lpszWorkstationName;

DWORD			dwState;

} WFSDEVSTATUS, *LPWFSDEVSTATUS;

The members of this structure are:

Field	Description

lpszPhysicalName	Pointer to the physical service name of the service that changed its state.

lpszWorkstationName	Pointer to the name of the workstation in which the logical service name is defined.

dwState	Specifies the new state of the physical device managed by the service as one of the following:

Value	Meaning

…

WFS_STAT_DEVOFFLINE	The device is offline (e.g. the operator has taken the device offline by turning a switch or pulling out the device).

### Clarifications for WFSOpen

Parameters	LPCSTR   lpszLogicalName

Points to a null-terminated string containing the pre-defined logical name of a service. It is a high level name such as "SYSJOURNAL1", "PASSBOOKPTR3" or "CASHDISP02," that is used by the XFS Manager and the Service Provider solely as a key to obtain the specific configuration information they need.

…

LPCSTR   lpszAppID

Points to a null-terminated string containing the application ID; the pointer may be NULL if the ID is not used. This ID may be used by services in a variety of ways; e.g. it is included in the SYSTEM_EVENT message for undeliverable events, to aid in finding system problems

### Clarifications for WFSAsyncOpen

Parameters	LPCSTR   lpszLogicalName

See WFSOpen.

…

LPCSTR   lpszAppID

Points to a null-terminated string containing the application ID. See WFSOpen.

### Clarifications for WFPOpen

Parameters

…

LPCSTR   lpszLogicalName

Points to a null-terminated string containing the pre-defined logical name of a service. It is a high level name such as "SYSJOURNAL1," "PASSBOOKPTR3" or "ATM02," that is used by the XFS Manager and the Service Provider as a key to obtain the specific configuration

…

LPCSTR   lpszAppID

Pointer to a null terminated string containing the application ID; the pointer may be NULL if the ID is not used.

### Clarifications for WFMOutputTraceData

Parameters	LPCSTR   lpszData

Pointer to a null-terminated string containing the trace data.

### Clarifications for WFMCreateKey

…

Be aware that when the WFMCreateKey is used for the first time and the hKey parameter is set to WFS_CFG_HKEY_XFS_ROOT then the existing registry structure will be migrated from HKEY_CLASSES_ROOT to HKEY_LOCAL_MACHINE. If any of the other values WFS_CFG_HKEY_MACHINE_XFS_ROOT, or WFS_CFG_HKEY_USER_DEFAULT_XFS_ROOT or WFS_CFG_CURRENT_USER_XFS_ROOT If either of the new values WFS_CFG_MACHINE_XFS_ROOT or WFS_CFG_USER_DEFAULT_XFS_ROOT are used then no migration will take place for this process. The assumption is that any process using the new key values will be doing its own migration. The reason migration does not always take place is that some applications will require access to both the old and new key roots so that they can migrate their non-CEN keys and values.

WFS_CFG_HKEY_XFS_ROOT is defined in XFS 2.x as HKEY_CLASSES_ROOT\WOSA/XFS_ROOT.

…

Parameters

…

LPCSTR   lpszSubKey

Pointer to a null-terminated string containing the name of the key to be created or opened.

### Clarifications for WFMDeleteKey

Parameters

…

LPCSTR   lpszSubKey

Pointer to a null-terminated string specifying the name of the key to be deleted.

### Clarifications for WFMDeleteValue

Parameters

…

LPCSTR   lpszValue

Pointer to a null-terminated string specifying the name of the value to be deleted.

### Clarifications for WFMOpenKey

Parameters

…

LPCSTR   lpszSubKey

Pointer to a null-terminated string containing the name of the key to be opened. If this parameter is NULL, or points to an empty string, the function opens another handle to the key identified by the hKey parameter (and does not close any previously opened handles).

### Clarifications for WFMQueryValue

Parameters

…

LPCSTR   lpszValueName

Pointer to a null-terminated string specifying the name of the value to be queried.

### Clarifications for WFMSetValue

Parameters

…

LPCSTR   lpszValueName

Pointer to a null-terminated string containing the name of the value being set. If a value with this name does not already exist in the specified key, it is added to the key.

LPCSTR   lpszData

Pointer to a buffer containing the data (a null-terminated character string) to be stored with the specified value name.

### Clarifications for XFSAPI.H

…

HRESULT extern WINAPI WFSOpen ( LPCSTR lpszLogicalName, HAPP hApp, LPCSTR lpszAppID, DWORD dwTraceLevel, DWORD dwTimeOut, DWORD dwSrvcVersionsRequired, LPWFSVERSION lpSrvcVersion, LPWFSVERSION lpSPIVersion, LPHSERVICE lphService);

…

HRESULT extern WINAPI WFSAsyncOpen ( LPCSTR lpszLogicalName, HAPP hApp, LPCSTR lpszAppID, DWORD dwTraceLevel, DWORD dwTimeOut, LPHSERVICE lphService, HWND hWnd, DWORD dwSrvcVersionsRequired, LPWFSVERSION lpSrvcVersion, LPWFSVERSION lpSPIVersion, LPREQUESTID lpRequestID);

### Clarifications for XFSADMIN.H

…

HRESULT extern WINAPI WFMOutputTraceData ( LPCSTR lpszData);

### Clarifications for XFSCONF.H

…

/******* Value of hKey *******************************************************/

#define     WFS_CFG_HKEY_XFS_ROOT               ((HKEY)1)

#define     WFS_CFG_HKEY_MACHINE_XFS_ROOT       ((HKEY)2)

#define     WFS_CFG_HKEY_USER_DEFAULT_XFS_ROOT  ((HKEY)3)

#define     WFS_CFG_CURRENT_USER_XFS_ROOT        ((HKEY)4)

// The following values are added for backwards compatibility reasons

#define     WFS_CFG_MACHINE_XFS_ROOT            WFS_CFG_HKEY_MACHINE_XFS_ROOT

#define     WFS_CFG_USER_DEFAULT_XFS_ROOT       WFS_CFG_HKEY_USER_DEFAULT_XFS_ROOT

…

HRESULT extern  WINAPI  WFMCreateKey ( HKEY hKey, LPCSTR lpszSubKey, PHKEY phkResult, LPDWORD lpdwDisposition);

HRESULT extern  WINAPI  WFMDeleteKey ( HKEY hKey, LPCSTR lpszSubKey);

HRESULT extern  WINAPI  WFMDeleteValue ( HKEY hKey, LPCSTR lpszValue );

…

HRESULT extern  WINAPI  WFMOpenKey ( HKEY hKey, LPCSTR lpszSubKey, PHKEY phkResult);

HRESULT extern  WINAPI  WFMQueryValue ( HKEY hKey, LPCSTR lpszValueName, LPSTR lpszData, LPDWORD lpcchData);

HRESULT extern  WINAPI  WFMSetValue ( HKEY hKey, LPCSTR lpszValueName, LPSTR lpszData, DWORD cchData);

### Clarifications for XFSSPI.H

HRESULT extern WINAPI WFPOpen ( HSERVICE hService, LPCSTR lpszLogicalName, HAPP hApp, LPCSTR lpszAppID, DWORD dwTraceLevel, DWORD dwTimeOut, HWND hWnd, REQUESTID ReqID, HPROVIDER hProvider, DWORD dwSPIVersionsRequired, LPWFSVERSION lpSPIVersion, DWORD dwSrvcVersionsRequired, LPWFSVERSION lpSrvcVersion);

## Printers and Scanners

Class Name		PTR

### Clarifications for WFS_INF_PTR_STATUS

fwMedia
Specifies the state of the print media (i.e. receipt, statement, passbook, etc.) as one of the following values. This field does not apply to journal printers:

Value	Meaning

WFS_PTR_MEDIAPRESENT	Media is in the print position, on the stacker or on the transport (i.e. a passbook in the parking station is not considered to be present). On devices with continuous paper supplies, this value is set when paper is under the print head. On devices with no sheet supply or individual sheet supplies, this value is set when paper/media is successfully inserted/loaded.

### Clarifications for WFS_INF_PTR_CAPABILITIES

Output Param	…

fwType
Specifies the type(s) of the physical device driven by the logical service, as a combination of the following flags:

Value	Meaning

WFS_PTR_TYPESCANNER	Device is a scanner with that may have printing capabilities.

### Clarifications for WFS_INF_PTR_QUERY_FIELD

Output Param	LPWFSFRMFIELD *lppFields;

fwOverflow
Specifies how an overflow of field data should be handled and can be one of the following values:

Value	Meaning

WFS_FRM_OVFWORDWRAP	If the field can hold more than one line the text is wrapped around. Wrapping is performed, where possible, by splitting the line on a space character or a hyphen character or any other character which is used to join two words together.

### Clarifications for WFS_CMD_PTR_READ_FORM

Output Param	..

lpszFields
Pointer to a series of "<FieldName>=<FieldValue>" strings, where each string is null-terminated with the entire field string terminating with two null characters. If the field is an index field, then the syntax of the string is instead "<FieldName>[<index>]=<FieldValue>", where <index> specifies the zero-based element of the index field. An empty list may be indicated by either a NULL pointer or a pointer to two consecutive null characters.

lpszUNICODEFields
Pointer to a series of "<FieldName>=<FieldValue>" UNICODE strings, where each string is null-terminated with the entire field string terminating with two null characters. If the field is an index field, then the syntax of the string is instead "<FieldName>[<index>]=<FieldValue>", where <index> specifies the zero-based element of the index field. An empty list may be indicated by either a NULL pointer or a pointer to two consecutive null characters.

### Clarifications for WFS_SRVE_PTR_MEDIATAKEN

Description	This event is sent when the media is taken from the exit slot following the completion of a successful eject operation or following a WFS_EXEE_PTR_MEDIAREJECTED event. For devices that do not physically move media, this event may also be generated when the media is taken from the device.

### Clarifications for WFS_SRVE_PTR_MEDIAAUTORETRACTED

Description	This event indicates when media has been automatically retracted by the device. Support for this event is indicated when the usAutoRetractPeriod field of the WFS_INF_PTR_CAPABILITIES output structure is non-zero. The event can be generated as the result of any command that presents media to the customer.

Event Param	LPWFSPTRMEDIARETRACTED lpMediaRetracted

typedef struct _wfs_ptr_media_retracted
	{
	WORD				wRetractResult;
	USHORT			usBinNumber;
	} WFSPTRMEDIARETRACTED, *LPWFSPTRMEDIARETRACTED;

wRetractResult
Specifies the result of the automatic retraction, as one of the following values:

Value	Meaning

WFS_PTR_AUTO_RETRACT_OK	The media was retracted successfully.

WFS_PTR_AUTO_RETRACT_MEDIAJAMMED
The media is jammed.

usBinNumber
Number of the retract bin the media was retracted to or zero if the media is retracted to the transport. This number has to be between one zero and the number of bins supported by this device. This value is zero if wRetractResult is WFS_PTR_AUTO_RETRACT_MEDIAJAMMED.

Comments	None.

### Clarifications for Definition Syntax

Other notes:

…

- A form and its optional subform that has multiple XFSFIELDs with the same fieldname is invalid. The WFS_ERR_PTR_FORMINVALID error will be returned if specified in the input to a command

- A form that has multiple XFSSUBFORMs with the same subformname is invalid. The WFS_ERR_PTR_FORMINVALID error will be returned if specified in the input to a command

- A form and its optional subform that has multiple XFSFRAMEs with the same framename is invalid. The WFS_ERR_PTR_FORMINVALID error will be returned if specified in the input to a command

### Clarifications for Form Definition

| XFSFORM |  | formname* |  |
| --- | --- | --- | --- |
| BEGIN |  |  |  |
|  | [ XFSFIELD | fieldname* | One field definition (as defined in the next section) for each field in the form. The fieldname within a form and its optional subforms must be unique. |
|  | BEGIN . . . END ] |  |  |
|  | [ XFSFRAME | framename* | One frame definition (as defined in the next section) for each frame in the form. The framename within a form and its optional subforms must be unique. |
|  | BEGIN . . . END ] |  |  |
|  | [ XFSSUBFORM | subformname* | One subform definition (as defined in the next section) for each subform in the form. The subformname within a form must be unique. |
|  | BEGIN . . . END ] |  |  |
| END |  |  |  |

### Clarifications for Subform Definition

| XFSSUBFORM |  | subformname* | The subformname within a form must be unique. |
| --- | --- | --- | --- |
| BEGIN |  |  |  |
| (required) | POSITION | X, | Horizontal position (relative to left side of form) |
|  | [ XFSFIELD | fieldname* | One field definition (as defined in the next section) for each field in the subform. The fieldname within a form and its optional subform must be unique. |
|  | BEGIN . . . END ] |  |  |
|  | [ XFSFRAME | framename* | One frame definition (as defined in the next section) for each frame in the subform. The framename within a form and its optional subform must be unique. |
|  | BEGIN . . . END ] |  |  |
| END |  |  |  |

### Clarifications for Field Definition

| XFSFIELD |  | fieldname* | The fieldname within a form and its optional subforms must be unique. |
| --- | --- | --- | --- |

## Identification Card Units

Class Name		IDC

### Clarifications for WFS_CMD_IDC_RETAIN_CARD

Events	In addition to the generic events defined in [Ref.1], the following events can be generated by this command:

Value	Meaning

WFS_EXEE_IDC_MEDIARETAINED	The card has been retained. This event is only fired if the command completes successfully (WFS_SUCCESS).

### Clarifications for WFS_CMD_IDC_CHIP_POWER

Description	This command handles the power actions that can be done on the chip.

For user chips, this command is only used after the chip has been contacted for the first time using the WFS_CMD_IDC_READ_RAW_DATA command. For contactless user chips, this command may be used to deactivate the contactless card communication.

…

Error Codes	In addition to the generic error codes defined in [Ref. 1], the following error codes can be generated by this command:

Value	Meaning

WFS_ERR_IDC_CHIPPOWERNOTSUPP	The specified action is not supported by the hardware device.

WFS_ERR_IDC_MEDIAJAM	The card is jammed (only applies to contact user chips). Operator intervention is required.

WFS_ERR_IDC_NOMEDIA	There is no card inside the device (may not apply for contactless user chips).

### Clarifications for C - Header File

/* Values of WFSIDCSTATUS.dwGuidLights [...]

WFSIDCCAPS.dwGuidLights [...],

WFSIDCSETGUIDLIGHT.wGuidLight */

#define     WFS_IDC_GUIDANCE_NOT_AVAILABLE      (0x00000000)

#define     WFS_IDC_GUIDANCE_OFF                (0x00000001)

/* The following value (WFS_IDC_GUIDANCE_ON) should NOT be used */

#define     WFS_IDC_GUIDANCE_ON                 (0x00000002)

#define     WFS_IDC_GUIDANCE_SLOW_FLASH         (0x00000004)

#define     WFS_IDC_GUIDANCE_MEDIUM_FLASH       (0x00000008)

#define     WFS_IDC_GUIDANCE_QUICK_FLASH        (0x00000010)

#define     WFS_IDC_GUIDANCE_CONTINUOUS         (0x00000080)

#define     WFS_IDC_GUIDANCE_RED                (0x00000100)

#define     WFS_IDC_GUIDANCE_GREEN              (0x00000200)

#define     WFS_IDC_GUIDANCE_YELLOW             (0x00000400)

#define     WFS_IDC_GUIDANCE_BLUE               (0x00000800)

#define     WFS_IDC_GUIDANCE_CYAN               (0x00001000)

#define     WFS_IDC_GUIDANCE_MAGENTA            (0x00002000)

#define     WFS_IDC_GUIDANCE_WHITE              (0x00004000)

#define     WFS_IDC_GUIDANCE_ENTRY              (0x00100000)

#define     WFS_IDC_GUIDANCE_EXIT               (0x00200000)

## Cash Dispensers

Class Name		CDM

### Clarifications for WFS_INF_CDM_STATUS

lppPositions
…

fwTransport
Supplies the state of the transport mechanism as one of the following values. The transport is defined as any area leading to or from the position:

### Clarifications for WFS_INF_CDM_CAPABILITIES

Output Param	..

fwRetractTransportActions
Specifies the actions which may be performed on items which have been retracted to the transport. If the device does not have the capability to retract items to the transport or move items from the transport this value will be WFS_CDM_NOTSUPP. This field will be a combination of the following flags:

Value	Meaning

WFS_CDM_PRESENT	The items may be presented.

WFS_CDM_RETRACT	The items may be retracted moved to a retract cash unit.

WFS_CDM_REJECT	The items may be retracted moved to a reject bin.

WFS_CDM_ITEMCASSETTE	The items may be retracted moved to the item cassettes, i.e. cassettes that can be dispensed from.

fwRetractStackerActions
Specifies the actions which may be performed on items which have been retracted to the stacker. If the device does not have the capability to retract items to the stacker or move items from the stacker this value will be WFS_CDM_NOTSUPP. Otherwise it will be a combination of the following flags:

Value	Meaning

WFS_CDM_PRESENT	The items may be presented.

WFS_CDM_RETRACT	The items may be retracted moved to a retract cash unit.

WFS_CDM_REJECT	The items may be retracted moved to a reject bin.

WFS_CDM_ITEMCASSETTE	The items may be retracted moved to the item cassettes, i.e. cassettes that can be dispensed from.

### Clarifications for WFS_INF_CDM_CASH_UNIT_INFO

Description		…

Counts

Item counts are typically based on software counts and therefore may not represent the actual number of items in the cash unit. Persistent values are maintained through power failures, open sessions, close session and system resets. If a cash unit is shared between the CDM and CIM device class, then CDM operations will result in count changes in the CIM cash unit structure and vice versa. All counts are reported consistently on both interfaces at all times.

On cash units that dispense items, if ulCount (on logical and physical cash units) reaches zero it will not decrement further but will remain at zero. When ulCount reaches zero no further dispense or denominate operations will be possible using that cash unit, unless the Service Provider provides a configuration option to continue using cash units when ulCount reaches zero. The default setting for any such configuration parameter must be to stop using the cash unit when this value reaches zero. If the Service Provider is configured such that the cash unit can still be used when ulCount reaches zero then WFS_CDM_STATCUEMPTY should not be generated when ulCount reaches zero, rather it should be generated when all physical cash units associated with the logical cash unit are physically empty. On recyclers, the Service Provider should not be configured to keep using the cash unit when ulCount is zero if the value in ulCount is used by any part of the application, as it may not be accurate. However, if the Service Provider is configured to keep using the cash unit when ulCount reaches zero, then the number of notes in the cash unit can be determined relative to ulInitialCount using ulDispensedCount, ulRetractedCount and the CIM ulCashInCount, e.g. Number of Notes = ulInitialCount – ulDispensedCount + ulRetractedCount + CIM::ulCashInCount.

On cash units that dispense items, ulCount on logical cash units is decremented when items are in customer access or successfully rejected. Therefore, items which are dispensed from the cash unit then removed from the device for other reasons (such as manual jam clearance) may mean that ulCount on logical cash units no longer reflects the number of items in the cash unit. The count of items in the cash unit should therefore be determined from other counts, e.g., ulCount on physical cash units or Number of Notes = ulInitialCount – ulDispensedCount + ulRetractedCount + CIM::ulCashInCount (if applicable).

Output Param	…

bAppLock
This field does not apply to reject or retract cash units. If this value is TRUE items cannot be dispensed from the cash unit. If this value is TRUE and the application attempts to dispense from the cash unit a WFS_EXEE_CDM_CASHUNITERROR event will be generated and a WFS_ERR_CDM_CASHUNITERROR code will be returned. This value is persistent.

### Clarifications for WFS_INF_CDM_PRESENT_STATUS

Description	This command is used to obtain the status of the most recent customer transaction from a specified output position.The items may have been dispensed and/or presented as a result of the WFS_CMD_CDM_PRESENT or A customer transaction starts with a WFS_CMD_CDM_DISPENSE command and completes when the items are presented to the customer, or the transaction is cancelled by calling a command such as WFS_CMD_CDM_REJECT. A customer transaction may include multiple WFS_CMD_CDM_DISPENSE commands if the capability fwMoveItems reports WFS_CDM_TOSTACKER. Commands during the customer transaction may cause this status to change.

Other commands which can dispense items such as WFS_CMD_CDM_TEST_CASH_UNITS do not update this status. This status is not updated as a result of any other command that can dispense/present items.

This value is persistent and is valid until the next time an attempt is made to present or dispense items to the customer transaction.The denominations reported by this command may not accurately reflect the operation if the cash units have been re-configured (e.g. if the values associated with a cash unit are changed, or new cash units are configured).

Input Param	LPWORD lpfwPosition;

lpfwPosition
Pointer to the required output position the items were presented or dispensed to as one of the following values:

Value	Meaning

WFS_CDM_POSNULL	The items were presented according to the default configuration.

WFS_CDM_POSLEFT	The items were presented to the left output position.

WFS_CDM_POSRIGHT	The items were presented to the right output position.

WFS_CDM_POSCENTER	The items were presented to the center output position.

WFS_CDM_POSTOP	The items were presented to the top output position.

WFS_CDM_POSBOTTOM	The items were presented to the bottom output position.

WFS_CDM_POSFRONT	The items were presented to the front output position.

WFS_CDM_POSREAR	The items were presented to the rear output position.

### Clarifications for WFS_CMD_CDM_DISPENSE

Description	...

If bPresent is set to TRUE and a shutter exists, then it will be implicitly controlled during the present operation, even if the bShutterControl capability is set to FALSE. The shutter will be closed when the user removes the items or the items are retracted.

Events	In addition to the generic events defined in [Ref. 1], the following events can be generated as a result of this command:

Value	Meaning

…

WFS_EXEE_CDM_INCOMPLETEDISPENSE	It has not been possible to dispense the entire denomination but part of the requested denomination has been dispensed, whether is on the intermediate stacker or in customer access. The return error code will be WFS_ERR_CDM_NOTDISPENSABLE.

### Clarifications for WFS_CMD_CDM_PRESENT

Description	This command will move items to the exit position for removal by the user. If a shutter exists, then it will be implicitly controlled during the present operation, even if the bShutterControl capability is set to FALSE. The shutter will be closed when the user removes the items or the items are retracted. If lpfwPosition points to WFS_CDM_POSNULL the position set in the WFS_CMD_CDM_DISPENSE command which caused these items to be dispensed will be used.

In the case where the shutter is unlocked but deliberately held shut, if the items could have been in customer access then a WFS_ERR_CDM_PRERRORITEMS error code will be returned.

When this command successfully completes the items are in customer access.

### Clarifications for WFS_CMD_CDM_RETRACT

Description	This command will retract items which may have been in customer access from an output position or from internal areas within the CDM. Retracted items will be moved to either a retract cash unit, a reject cash unit, item cash units, the transport or the intermediate stacker. After the items are retracted the shutter is closed automatically, even if the bShutterControl capability is set to FALSE.

### Clarifications for WFS_CMD_CDM_END_EXCHANGE

Error Codes	In addition to the generic error codes defined in [Ref. 1], the following error codes can be generated by this command:

Value	Meaning

WFS_ERR_CDM_CASHUNITERROR	A problem occurred with a cash unit A WFS_EXEE_CDM_CASHUNITERROR event will be posted with the details. A cash unit problem occurred that meant no cash units could be updated. One or more WFS_EXEE_CDM_CASHUNITERROR events will be sent with the details.

Events	In addition to the generic events defined in [Ref. 1], the following events can be generated by this command:

Value	Meaning

WFS_EXEE_CDM_CASHUNITERROR	A cash unit caused an error problem occurred that meant no cash units could be updated. One or more WFS_EXEE_CDM_CASHUNITERROR events will be sent with the details.

### Clarifications for WFS_CMD_CDM_CALIBRATE_CASH_UNIT

Input Param	LPWFSCDMCALIBRATE lpCalibrateIn;

typedef struct _wfs_cdm_calibrate
	{
	USHORT				usNumber;
	USHORT				usNumOfBills;
	LPWFSCDMITEMPOSITION		*lpPosition;
	} WFSCDMCALIBRATE, *LPWFSCDMCALIBRATE;

..

lpPosition
Specifies where the dispensed items should be moved to. For a description of the WFSCDMITEMPOSITION structure see section Error: Reference source not found.

This parameter is a pointer to a pointer to WFSCDMITEMPOSITION structure.

### Clarifications for WFS_CMD_CDM_PREPARE_DISPENSE

Error Codes	In addition to the generic error codes defined in [Ref. 1], the following error codes can be generated by this command:

Value	Meaning

WFS_ERR_CDM_EXCHANGEACTIVE	The CDM is in an exchange state.

### Clarifications for WFS_CMD_CDM_RESET

Input Param	…

fwOutputPosition
If usNumber is zero and lpRetractArea is NULL then this field defines the output position to which items are to be moved. In all other cases this field will be ignored. Possible values areThe output position to which items are to be moved. If the usNumber is non-zero then this field will be ignored. The value is specified as one of the following values:

Value	Meaning

WFS_CDM_POSNULL	The default configuration.

WFS_CDM_POSOUTLEFT	The left output position.

WFS_CDM_POSOUTRIGHT	The right output position.

WFS_CDM_POSOUTCENTER	The center output position.

WFS_CDM_POSOUTTOP	The top output position.

WFS_CDM_POSOUTBOTTOM	The bottom output position.

WFS_CDM_POSOUTFRONT	The front output position.

WFS_CDM_POSOUTREAR	The rear output position.

### Clarifications for WFS_EXEE_CDM_INCOMPLETEDISPENSE

Description	This execute event is generated during WFS_CMD_CDM_DISPENSE when it has not been possible to dispense the entire denomination but part of the requested denomination is on the intermediate stacker or in customer access when not all of the items specified in a WFS_CMD_CDM_DISPENSE operation could be dispensed. Some of the items have been dispensed. If the device has no intermediate stacker then the items that were dispensed will be in customer access.

Event Param	LPWFSCDMDENOMINATION lpDenomination;

lpDenomination
The WFSCDMDENOMINATION structure is defined in the documentation of the command WFS_CMD_CDM_DENOMINATE. Note that in this case the values in this structure report the amount and number of each denomination that are in customer access or on the intermediate stacker. WFS_INF_CDM_PRESENT_STATUS can be used to determine whether the items are in customer accesshas actually been dispensed.

Comments	None.

### Clarifications for C - Header File

#define     WFS_CDM_GUIDANCE_NOT_AVAILABLE            (0x00000000)

#define     WFS_CDM_GUIDANCE_OFF                      (0x00000001)

#define     WFS_CDM_GUIDANCE_SLOW_FLASH               (0x00000004)

#define     WFS_CDM_GUIDANCE_MEDIUM_FLASH             (0x00000008)

#define     WFS_CDM_GUIDANCE_QUICK_FLASH              (0x00000010)

#define     WFS_CDM_GUIDANCE_CONTINUOUS               (0x00000080)

#define     WFS_CDM_GUIDANCE_RED                      (0x00000100)

#define     WFS_CDM_GUIDANCE_GREEN                    (0x00000200)

#define     WFS_CDM_GUIDANCE_YELLOW                   (0x00000400)

#define     WFS_CDM_GUIDANCE_BLUE                     (0x00000800)

#define     WFS_CDM_GUIDANCE_CYAN                     (0x00001000)

#define     WFS_CDM_GUIDANCE_MAGENTA                  (0x00002000)

#define     WFS_CDM_GUIDANCE_WHITE                    (0x00004000)

#define     WFS_CDM_GUIDANCE_ENTRY                    (0x00100000)

#define     WFS_CDM_GUIDANCE_EXIT                     (0x00200000)

/* Values of WFSCDMSTATUS.dwGuidLights [...]

WFSCDMCAPS.dwGuidLights [...] */

#define     WFS_CDM_GUIDANCE_NOT_AVAILABLE            (0x0000)

## Personal Identification Number Keypads (PIN Pads)

Class Name		PIN

### Clarifications for Section 3. References

Added reference:

38. PCI PIN Transaction Security (PTS) Point of Interaction (POI) version 6.1

### Clarifications for WFS_INF_PIN_CAPABILITIES

Output Param	…

fwKeyCheckModes
Specifies the key check modes that are supported to check the correctness of an imported key value. The modes available for each key may depend on security requirements of the algorithm (for example, see [Ref. 38]). The algorithm (i.e. DES, 3DES, AES, SM4) and use is determined by the algorithm of the key being checked and security requirements. If the key size is larger than the algorithm block size, then only the first block will be used. It can be a combination of the following flags:

Value	Meaning

WFS_PIN_KCVSELF	The key check value (KCV) is created by an encryption of the key with itself. For a double length key the KCV is generated using 3DES encryption using the first half of the key as the source data for the encryption.

WFS_PIN_KCVZERO	The key check value (KCV) is created by encrypting a zero value with the key.

### Clarifications for WFS_CMD_PIN_CRYPT

Input Param	…

wMode
If MACing then this parameter will be ignored, otherwise this parameter sSpecifies the modewhether to encrypt or decrypt, values are one of the following:

Value	Meaning

WFS_PIN_MODEENCRYPT	Encrypt with key.

WFS_PIN_MODEDECRYPT	Decrypt with key.

WFS_PIN_MODERANDOM	An 8 byte random value shall be returned (in this case all the other input parameters are ignored).

### Clarifications for WFS_CMD_PIN_GET_DATA

Description	This function is used to return keystrokes entered by the user. It will automatically set the PIN pad to echo characters on the display if there is a display. For each keystroke an execute notification event WFS_EXEE_PIN_KEY is sent in order to allow an application to perform the appropriate display action (i.e. when the PIN pad has no integrated display).

The WFS_EXEE_PIN_ENTERDATA event will be generated when the PIN pad is ready for the user to start entering data.

When the maximum number of digits is entered and the flag bAutoEnd is true, or a terminate key is pressed after the minimum number of digits is entered, the command completes. If the <Cancel> key is a terminator key and is pressed, the command will complete successfully even if the minimum number of digits has not been entered.

Terminating FDKs can have the functionality of <Enter> (terminates only if minimum length has been reached) or <Cancel> (can terminate before minimum length is reached). The configuration of this functionality is vendor specific.

### Clarifications for WFS_CMD_PIN_GET_PINBLOCK

Input Param	…

lpsCustomerData
The customer data should be an ASCII string. Used for ANSI, ISO-0 and, ISO-1 and ISO-3 algorithm to build the formatted PIN. For ANSI and, ISO-0 and ISO-3 the PAN (Primary Account Number, without the check number) is supplied, for ISO-1 a ten digit transaction field is required. If not used a NULL is required.

bPadding
Specifies the padding character. The valid range is 0x00 to 0x0F. Only the least significant nibble is used. This field is ignored for PIN block formats with fixed, sequential or random padding.

### Clarifications for WFS_CMD_CDM_RESET

Description	Sends a service reset to the Service Provider. This command may trigger a self-test, for example, initializing memory, checking device state, etc. For the details of any self-test performed, vendor specific documentation may have to be consulted.

### Clarifications for WFS_CMD_PIN_IMPORT_RSA_ENCIPHERED_PKCS7_KEY_EX

Output Param	…

dwRSAKeyCheckMode
Defines algorithm/method used to generate the public key check value/thumb print. The check value can be used to verify that the public key has been imported correctlyDefines the algorithm used to generate the signature contained in the message (lpxRSAData) sent to the host (see section 8.2.2 step 2c). It can be one of the following flags:

### Clarifications for C - Header File

..

/* Values of WFSPINSTATUS.dwGuidLights [...]

WFSPINCAPS.dwGuidLights [...]

*/

#define     WFS_PIN_GUIDANCE_NOT_AVAILABLE          (0x00000000)

#define     WFS_PIN_GUIDANCE_OFF                    (0x00000001)

/* The following value (WFS_PIN_GUIDANCE_ON) should NOT be used */

#define     WFS_PIN_GUIDANCE_ON                     (0x00000002)

#define     WFS_PIN_GUIDANCE_SLOW_FLASH             (0x00000004)

#define     WFS_PIN_GUIDANCE_MEDIUM_FLASH           (0x00000008)

#define     WFS_PIN_GUIDANCE_QUICK_FLASH            (0x00000010)

#define     WFS_PIN_GUIDANCE_CONTINUOUS             (0x00000080)

#define     WFS_PIN_GUIDANCE_RED                    (0x00000100)

#define     WFS_PIN_GUIDANCE_GREEN                  (0x00000200)

#define     WFS_PIN_GUIDANCE_YELLOW                 (0x00000400)

#define     WFS_PIN_GUIDANCE_BLUE                   (0x00000800)

#define     WFS_PIN_GUIDANCE_CYAN                   (0x00001000)

#define     WFS_PIN_GUIDANCE_MAGENTA                (0x00002000)

#define     WFS_PIN_GUIDANCE_WHITE                  (0x00004000)

#define     WFS_PIN_GUIDANCE_ENTRY                  (0x00100000)

#define     WFS_PIN_GUIDANCE_EXIT                   (0x00200000)

### Clarifications for Remote Key Exchange

Host

The Host starts the Key Exchange process.

The Host sends the  Signed Key Transport Key message to the ATM.

The Host receives the message and verifies the message and checks to make sure it is talking to to the right ATM.

PIN

The PIN generates random number and sends it to the host

The PIN verifies the messages and if validate stores the key. The PIN then sends a message back to the Host.

RATM

Sign(SKHost)[RHost||RATM||IATM||RSACrypt(PKATM)[IHost||KKTK]]

Sign(SKATM)[RHost||RATM||IHost]

Request RATM

### Clarifications for Replace Certificate

Host

Host wants to take CA duties, sends new Certificate

PIN

The PIN verifies the message, if valid the PIN stores the new CA.

The PIN then sends the thumbprint.

Sign(SKHost)[CertCA]

TPATM

### Clarifications for Luxemburg Protocol

Comments	Luxembourg encryption commands defined in the following paragraphs will return the generic error PROT_LUX_ERR_INVALID_DATA when the input data is invalid.
Note that since the introduction of the error codes for the Luxemburg Protocol, they have been redefined in the header file as positive values. This is to correct the original oversight of being defined as negative values which cannot be meaningfully returned in the WORD wResult output parameter. They have therefore been redefined as positive values in such a way that existing and future implementations which type cast them to an unsigned type will not be impacted.

### Clarifications for Luxemburg-specific Header File

..

#define PROT_LUX_SUCCESS                     (0)

#define PROT_LUX_ERR_INVALID_CMD             (USHRT_MAX-(PROT_LUX_RESULT_OFFSET + 1))

#define PROT_LUX_ERR_INVALID_DATA            (USHRT_MAX-(PROT_LUX_RESULT_OFFSET + 21))

#define PROT_LUX_ERR_INVALID_KEY             (USHRT_MAX-(PROT_LUX_RESULT_OFFSET + 32))

/* values of PROTLUXLOADAPPKEYOUT.wResult */

/* values of PROTLUXCHECKMACOUT.wResult   */

#define PROT_LUX_ERR_VERIFICATION_FAILED     (USHRT_MAX -(PROT_LUX_RESULT_OFFSET + 43))

/* values of PROTLUXPINBLOCKOUT.wResult */

#define PROT_LUX_ERR_PIN_FORMAT_LENGTH       (USHRT_MAX -(PROT_LUX_RESULT_OFFSET + 54))

## Check Reader/Scanner

Class Name		CHK

## Depository Unit

Class Name		DEP

## Text Terminal Unit

Class Name		TTU

## Sensors and Indicators Units

Class Name		SIU

### Clarifications for Enhanced Audio Controller Overview

Auto Mode

In auto mode, when a consumer activates the a Privacy Device, the audio is automatically directed to the that Privacy Device and the audio is no longer sent to the speakers. When the Privacy Device is deactivated, the audio is redirected to the speakers. If more than one Privacy Device has been activated, audio is not redirected to the speakers until all Privacy Devices have been deactivated. The following state diagram completely describes the behavior of the device in auto mode.

Semi-Auto Mode

The dashed-line transitions are caused by application calls to WFS_CMD_SIU_SET_PORTS or WFS_CMD_SIU_SET_AUXILIARY for the WFS_SIU_ENHANCEDAUDIOCONTROL auxiliary with values of WFS_SIU_PRIVATEAUDIO_SEMI_AUTO or WFS_SIU_PUBLICAUDIO_SEMI_AUTO.

### Clarifications for WFS_INF_SIU_STATUS

Output Param	…

fwAuxiliaries [WFS_SIU_ENHANCEDAUDIOCONTROL]
Specifies the state of the Enhanced Audio Controller. The Enhanced Audio Controller controls how private and public audio are broadcast when the headset is inserted into/removed from the audio jack and when the handset is off-hook/on-hook. In the following, Privacy Device is used to refer to either the headset or handset. The Enhanced Audio Controller state is specified as one of the following flags:

Value	Meaning

WFS_SIU_PRIVATEAUDIO_AUTO	The Enhanced Audio Controller is in auto mode and is in the private state (i.e. audio will be played only through a connected Privacy Device). In private mode, no audio is transmitted through the speakers. When a Privacy Device is deactivated (headset disconnected/handset on-hook), the device will go to the public state. Where there is more than one Privacy Device, the device will go to the public state only when all Privacy Devices have been deactivated.

### Clarifications for C - Header file

/*=================================================================*/

/* SIU Info Command Structures and variables */

/*=================================================================*/

typedef struct wfs_siu_get_startup_time

{

WORD            wMode;

LPSYSTEMTIME    lpStartTime;

} WFSSIUGETSTARTUPTIME, *LPWFSSIUGETSTARTUPTIME;

/*=================================================================*/

/* SIU Execute Command Structures */

/*=================================================================*/

typedef struct wfs_siu_set_startup_time

{

WORD            wMode;

LPSYSTEMTIME    lpStartTime;

} WFSSIUSETSTARTUPTIME, *LPWFSSIUSETSTARTUPTIME;

## Vendor Dependent Mode

Class Name		VDM

### Clarifications for WFS_CMD_VDM_ENTER_MODE_ACK

Description	This command is issued by a registered application as an acknowledgement to the WFS_SRVE_VDM_ENTER_MODE_REQ event and it indicates that the application is ready for the system to enter Vendor Dependent Mode. All registered applications (including the application that issued the request to enter Vendor Dependent Mode) must respond before Vendor Dependent Mode will be entered. Completion of this command is immediate.

Note: Applications must be prepared to allow the Vendor Dependent Application to display on the active interface. This means that applications should no longer try to be the foreground or topmost window to ensure that the Vendor Dependent Application is visible.

### Clarifications for WFS_SRVE_VDM_INTERFACE_ CHANGED

Description	This service event is used to indicate that the required interface has changed. This can be as a result of a WFS_CMD_VDM_SET_ACTIVE_INTERFACE command, or when the active interface is changed through vendor dependent means while in VDM. The wActiveInterface field of the WFSVDMACTIVEINTERFACE structure indicates which interface has been selected.

Note: Applications must be prepared to allow the Vendor Dependent Application to display on the active interface. This means that applications should no longer try to be the foreground or topmost window to ensure that the Vendor Dependent Application is visible.

## Cameras

Class Name		CAM

### Clarifications for WFS_INF_CAM_STATUS

Output Param	..

fwMedia […]
Specifies the state of the recording media of the cameras. A number of indexes are defined below. The maximum fwMedia index is WFS_CAM_CAMERAS_MAX. For a device which stores pictures on a hard disk drive or other general-purpose storage, the value of the fwMedia field should be WFS_CAM_MEDIANOTSUPP.

## Alarms

Class Name		ALM

No clarifications available.

## Card Embossing Unit

Class Name		CEU

## Cash In Module

Class Name		CIM

### Clarifications for WFS_INF_CIM_STATUS

Output Param	…

fwIntermediateStacker
Supplies the state of the intermediate stacker as one of the following values:

Value	Meaning

WFS_CIM_ISEMPTY	The intermediate stacker is empty.

WFS_CIM_ISNOTEMPTY	The intermediate stacker is not empty.

WFS_CIM_ISFULL	The intermediate stacker is full. This may also be reported during a cash-in transaction where a limit specified by WFS_CMD_CIM_SET_CASH_IN_LIMIT has been reached.

lppPositions
…

fwTransport
Supplies the state of the transport mechanism as one of the following values. The transport is defined as any area leading to or from the position:

### Clarifications for WFS_INF_CIM_CAPABILITIES

Output Param	…

fwExchangeType
Specifies the type of cash unit exchange operations supported by the CIM. Values are a combination of the following flags:

Value	Meaning

WFS_CIM_EXBYHAND	The CIM supports manual replenishment either by emptying the cash unit by hand or by replacing the cash unit.

WFS_CIM_EXTOCASSETTES	The CIM supports moving items from the bill cash units to the replenishment cash unit to the bill cash units.

WFS_CIM_CLEARRECYCLER	The CIM supports the emptying of recycle cash units.

WFS_CIM_DEPOSITINTO	The CIM supports moving items from the deposit entrance to the bill cash units.

lpszExtra
The parameter that reports how notes are classified and handled is reported in lpszExtra as follows. If level 2/3 notes are not to be returned to the customer by these rules, they will not be returned regardless of whether their specific note type is configured to not be accepted by WFS_CMD_CIM_CONFIGURE_NOTETYPES:

Comments	…

| bShutter | bShutterControl | WFSCIMPOSCAPS .bPresentControl | Description |
| --- | --- | --- | --- |
| TRUE | TRUE | TRUE | Service Provider implicitly opens the shutter, presents items and closes the shutter when all items are taken. |
| TRUE | TRUE | FALSE | Service Provider implicitly opens the shutter for input. Application required to present items using WFS_CMD_CIM_PRESENT_MEDIA. |
| TRUE | FALSE | TRUE | Application is required to present items using WFS_CMD_CIM_OPEN_SHUTTER and then call WFS_CMD_CIM_CLOSE_SHUTTER when all items are taken. |
| TRUE | FALSE | FALSE | Application is required to present items either by using WFS_CMD_CIM_PRESENT_MEDIA, or alternatively, by using, WFS_CMD_CIM_OPEN_SHUTTER and then WFS_CMD_CIM_CLOSE_SHUTTER when all items are taken. |
| FALSE | TRUE | TRUE | Service Provider implicitly opens the shutter, presents items and closes the shutter when all items taken. |
| FALSE | TRUE | FALSE | Service Provider implicitly opens the shutter for input. Application required to present items using WFS_CMD_CIM_PRESENT_MEDIA. |
| FALSE | FALSE | TRUE | Not Supported. |
| FALSE | FALSE | FALSE | Application required to present items using WFS_CMD_CIM_PRESENT_MEDIA. |

### Clarifications for WFS_INF_CIM_CASH_UNIT_INFO

Output Param	…

usStatus
Describes the status of the cash unit as one of the following values:

Value	Meaning

…

WFS_CIM_STATCULOW	The cash unit is almost empty (i.e. reached or below the threshold defined by ulMinimum). This value is only reported for CDM specific cash units which can dispense media items.(fwType == WFS_CIM_TYPECDMSPECIFIC) It is not mandatory to report this for recycle cash units (fwType == WFS_CIM_TYPERECYCLING).

…

lppPhysical
Pointer to an array of pointers to WFSCIMPHCU structures:

…

usPStatus
Supplies the status of the physical cash unit as one of the following values:

Value	Meaning

…

WFS_CIM_STATCULOW	The cash unit is almost empty. This value is only reported for CDM specific cash units which can dispense media items. It is not mandatory to report this for recycle cash units (fwType == WFS_CIM_TYPERECYCLING) (fwType == WFS_CIM_TYPECDMSPECIFIC).

…

### Clarifications for WFS_INF_CIM_BANKNOTE_TYPES

Output Param	…

bConfigured
Specifies whether or not the banknote reader recognizes this note type. If TRUE the banknote reader will accept this note type during a cash-in operation, if FALSE the banknote reader will refuse this note type unless it must be retained by legislative note handling rules.

### Clarifications for WFS_INF_CIM_CASH_IN_STATUS

Description	This command is used to get information about the status of the last currently active cash-in transaction or in the case where no cash-in transaction is active the status of the most recently ended Cash-In transaction.
This value is persistent and is valid until the next WFS_CMD_CIM_CASH_IN_START.

Input Param	None.

Output Param	LPWFSCIMCASHINSTATUS	lpCashInStatus;

typedef struct _wfs_cim_cash_in_status
{
WORD	wStatus;
USHORT	usNumOfRefused;
LPWFSCIMNOTENUMBERLIST	lpNoteNumberList;
LPSTR	lpszExtra;
} WFSCIMCASHINSTATUS, * LPWFSCIMCASHINSTATUS;

wStatus
Status of the currently active or most recently ended Cash-In transaction. Possible values are:

Value	Meaning

WFS_CIM_CIOK	The Cash-In transaction is complete and has ended with a WFS_CMD_CIM_CASH_IN_END command call.

WFS_CIM_CIROLLBACK	The Cash-In transaction was rolled back has ended with a WFS_CMD_CIM_CASH_IN_ROLLBACK command call.

WFS_CIM_CIACTIVE	There is a Cash-In transaction active. See the WFS_CMD_CIM_CASH_IN_START command description for a definition of an active cash-in transaction.

WFS_CIM_CIRETRACT	The Cash-In transaction ended with the items being retracted with a WFS_CMD_CIM_RETRACT command call, or a retract command call on a compound device class.

WFS_CIM_CIUNKNOWN	The state of the Cash-In transaction is unknown.
The cash-in transaction ended with a WFS_CMD_CIM_RESET command call, or a reset command call on a compound device class.

usNumOfRefused
Specifies the number of items refused during the currently active or most recently ended Cash-In transaction period.

lpNoteNumberList
List of banknote types that were inserted, identified and accepted during the currently active or most recently ended cash-in transaction period. The WFSCIMNOTENUMBER .ulCount value within this structure is the count of items of identified and accepted notes during the cash-in transaction period. If notes items have been rolled back (wStatus is WFS_CIM_CIROLLBACK) they will be included in this list. If wStatus is WFS_CIM_CIRETRACT or WFS_CIM_CIRESET then identified and accepted items moved to Cash-In or Recycle cash units are included in this list, but items moved to the Retract or Reject cash units are not included. For a description of the WFSCIMNOTENUMBERLIST structure see the definition of the command WFS_INF_CIM_CASH_UNIT_INFO.

If a note handling standard is supported then lpNoteNumberList includes any level 2 or level 3 notes.

### Clarifications for WFS_INF_CIM_GET_P6_SIGNATURE

Output Param	..

usIndex
Specifies the index (zero to usNumOfSignatures-1) of the required signature.

Note: Signatures may be returned in any order; there is no implied relationship between this index and the order in which items are reported in the lpNoteNumberList in WFS_INF_CIM_GET_P6_INFO.

### Clarifications for WFS_INF_CIM_ GET_ITEMS_INFO

Output Param	..

lpszImageFileName
This field contains the serial number of the item as a Unicode string. A '?' character (0x003F) is used to represent any serial number character that cannot be recognized. If no serial number is available or has not been requested then lpszSerialNumber is NULL.

The application is responsible for the use and management of this file. For example, the application can transfer the image files to a directory which is managed by the application.

### Clarifications for WFS_CMD_CIM_CASH_IN_START

Description	Before initiating a cash-in operation, an application must issue the WFS_CMD_CIM_CASH_IN_START command to begin a cash-in transaction. During a cash-in transaction any number of WFS_CMD_CIM_CASH_IN commands may be issued. The transaction is ended when either a WFS_CMD_CIM_CASH_IN_ROLLBACK, WFS_CMD_CIM_CASH_IN_END, WFS_CMD_CIM_RETRACT or WFS_CMD_CIM_RESET command is sent. Where WFSCIMCAPS.bShutterControl == FALSE this command precedes any explicit operation of the shutters.

…

A hardware failure during the cash-in transaction does not reset the note number list information; instead the note number list information will include items that could be accepted and identified up to the point of the hardware failure.

Input Param	..

bUseRecycleUnits
Specifies whether or not the recycle cash units should be used when items are cashed in on a successful WFS_CMD_CIM_CASH_IN_END command for money cashed in during the transaction period. This parameter will be ignored if there are no recycle cash units or the hardware does not support this.

### Clarifications for WFS_CMD_CIM_CASH_IN

Description	...

If bShutterControl is TRUE, it is also possible that a device may return refused notes in multiple subsequent bunches.  In this case, the WFS_CMD_CIM_CASH_IN command will not complete until the final bunch has been presented and the WFS_SRVE_CIM_ITEMSPRESENTED has been generated.

…

Error Codes	In addition to the generic error codes defined in [Ref. 1], the following error codes can be generated by this command:

Value	Meaning

…

WFS_ERR_CIM_TOOMANYITEMS	There were too many items inserted previously. The cash-in stacker is full at the beginning of this command. This may also be reported where a limit specified by WFS_CMD_CIM_SET_CASH_IN_LIMIT has already been reached at the beginning of this command.

### Clarifications for WFS_CMD_CIM_OPEN_SHUTTER

Description	This command opens the shutter.

In cases where multiple bunches are to be returned under explicit shutter control and the first bunch has already been presented and taken and the output position is empty, this command moves the next bunch to the output position before opening the shutter. This does not apply if the output position is not empty, for example if items had been re-inserted or dropped back into the output position as the shutter closed.

…

Events	In addition to the generic events defined in [Ref. 1], the following events can be generated as a result of this command:

Value	Meaning

WFS_SRVE_CIM_ITEMSTAKEN	The items have been removed by the user. This event is only generated if the bItemsTakenSensor field returned in the capabilities information is TRUE.

WFS_SRVE_CIM_ITEMSINSERTED	Items have been inserted by the user.

WFS_SRVE_CIM_SHUTTERSTATUSCHANGED
The shutter status has changed.

WFS_SRVE_CIM_ITEMSPRESENTED	Items have been presented to the user to be taken.

### Clarifications for WFS_CMD_CIM_RESET

Output Param	…

fwOutputPosition
If usNumber is zero and lpRetractArea is NULL then this field defines the output position to which items are to be moved. In all other cases this field will be ignored. Possible values areThe output position to which items are to be moved. If the usNumber is non-zero or if lpRetractArea indicates WFS_CIM_RA_BILLCASSETTES then this field must be zero. The value is set to one of the following values:

Value	Meaning

WFS_CIM_POSNULL	Take the default configuration.

WFS_CIM_POSOUTLEFT	Move items to the left output position.

WFS_CIM_POSOUTRIGHT	Move items to the right output position.

WFS_CIM_POSOUTCENTER	Move items to the center output position.

WFS_CIM_POSOUTTOP	Move items to the top output position.

WFS_CIM_POSOUTBOTTOM	Move items to the bottom output position.

WFS_CIM_POSOUTFRONT	Move items to the front output position.

WFS_CIM_POSOUTREAR	Move items to the rear output position.

### Clarifications for WFS_CMD_CIM_CONFIGURE_NOTETYPES

Description	This command is used to configure the note types the banknote reader will should recognize accept during cash-in. All note types the banknote reader has toshould recognize accept must be given in the input structure. If an unknown note type is given the error code WFS_ERR_UNSUPP_DATA will be returned.

### Clarifications for WFS_CMD_CIM_REPLENISH

Events	In addition to the generic events defined in [Ref. 1], the following events can be generated by this command:

Value	Meaning

WFS_USRE_CIM_CASHUNITTHRESHOLD	A threshold condition has occurred in one of the cash units.

WFS_EXEE_CIM_CASHUNITERROR	A problem occurred with a cash unit.

WFS_EXEE_CIM_NOTEERROR	An item detection error has occurred.

WFS_EXEE_CIM_INPUT_P6	Level 2 and / or level 3 notes are detected during this operation.

WFS_EXEE_CIM_INFO_AVAILABLE	Information is available for items detected during the cash processing operation.

WFS_EXEE_CIM_INCOMPLETEREPLENISH
If this command fails with an error code (not WFS_SUCCESS) but some items have been moved, then the details will be reported with this event. This event can only occur once per command.

### Clarifications for WFS_CMD_CIM_SET_CASH_IN_LIMIT

Input Param	LPWFSCIMCASHINLIMIT lpCashInLimit;

Pointer to the WFSCIMCASHINLIMIT structure. This cash-in limit structure can be used to limit the items that can be accepted during the cash-in operationtransaction. The limit set does not include counterfeit or suspected counterfeit items which may be detected during such a cash-in operationtransaction. If the lpCashInLimit field is set to a NULL pointer there is no specific amount/number of items limit for the next cash-in operationtransaction. Note that the cash-in limit set by this command may itself be limited by the physical cash-in limitation of the device.

If one or more limit conditions have been set by this command, the limit reached during the WFS_CMD_CIM_CASH_IN commandcash-in operation will be reported in the lpusReason field of the WFS_EXEE_CIM_INPUTREFUSE event.

typedef struct _wfs_cim_cash_in_limit
	{
	ULONG					ulTotalItemsLimit;
	LPWFSCIMAMOUNTLIMIT		lpAmountLimit;
	} WFSCIMCASHINLIMIT, *LPWFSCIMCASHINLIMIT;

ulTotalItemsLimit
If set to a non-zero value, specifies a limit on the total number of items to be accepted during the cash-in operationtransaction. If set to a zero value, this limitation will not be performed.

This limitation can only be used if WFS_CIM_LIMITBYTOTALITEMS is specified in the fwCashInLimit field of the WFS_INF_CIM_CAPABILITIES command. If however this is specified but not supported the WFS_ERR_UNSUPP_DATA error will be returned and no limit will be set.

lpAmountLimit
Pointer to the WFSCIMAMOUNTLIMIT structure. If set to a NULL pointer this limitation will not be performed. For CIM devices which can accept more than one currency this limit can only be applied to one currency for each cash-in operationtransaction.

This limitation can only be used if WFS_CIM_LIMITBYAMOUNT is specified in the fwCashInLimit field of the WFS_INF_CIM_CAPABILITIES command. If however this is specified but not supported the WFS_ERR_UNSUPP_DATA error will be returned and no limit will be set.

typedef struct _wfs_cim_amount_limit
	{
	CHAR					cCurrencyID[3];
	ULONG					ulAmount;
	} WFSCIMAMOUNTLIMIT, *LPWFSCIMAMOUNTLIMIT;

cCurrencyID
Currency identifier in ISO 4217 format [Ref. 2]. This must not be three ASCII 0x20 characters.

ulAmount
If set to a non-zero value, specifies a limit on the total amount of the cash-in operationtransaction. This value is expressed in minimum dispense units (see section WFS_INF_CIM_CURRENCY_EXP). If set to a zero value, this limitation will not be performed.

### Clarifications for WFS_CMD_CIM_PRESENT_MEDIA

Description	This command opens the shutter and presents items to be taken by the customer. The shutter is automatically closed after the media is taken. The command can be called after a WFS_CMD_CIM_CASH_IN, WFS_CMD_CIM_ROLLBACK, WFS_CMD_CIM_RESET or WFS_CMD_CIM_CREATE_P6_SIGNATURE command and can be used with explicit and implicit shutter control.

### Clarifications for WFS_SRVE_CIM_ITEMSTAKEN

Description	This service event specifies that items presented to the user have been taken. This event may be generated at any time.

### Clarifications for WFS_EXEE_CIM_INPUTREFUSE

Event Param	LPUSHORT lpusReason;

lpusReason
Pointer to an USHORT holding the reason for refusing a part of the amount. Possible values are:

Value	Meaning

WFS_CIM_DEPOSITFAILURE	A deposit has failed for a reason not covered by the other reasons and the failure is not a fatal hardware problem, for example failing to pick an item from the input area.

WFS_CIM_INVALIDBUNCH	Recognition of the items did not take place. The bunch of notes presented inserted is invalid, e.g. it is too large or was presented inserted incorrectly.

## Clarifications for ATM Cash-In Transaction Flow - Application Guidelines

### 8.16 Multiple Bunch Timeout Handling

The following sections describe flows where the Service Provider could potentially present refused items in multiple bunches during the WFS_CMD_CIM_CASH_IN command. As the WFS_CMD_CIM_CASH_IN timeout (dwTimeout parameter in WFSAsyncExecute or WFSExecute) may elapse before the last bunch is presented, resulting in a WFS_ERR_TIMEOUT in the completion event, it is recommended that the application take control by specifying a long dwTimeout and use timers to allow sufficient time for user interaction before cancelling the command. dwTimeout should be set sufficiently long to allow for any scenario; it could be set to WFS_INDEFINITE_WAIT as the command would be explicitly cancelled by the application if timers elapse.

Each flow covers the following cases:

- bShutterControl == TRUE, bItemsInsertedSensor == TRUE, bItemsTakenSensor == TRUE, bPresentControl == TRUE

### No Items Inserted

In this flow, the user does not insert items within the required time, therefore the application cancels the WFS_CMD_CIM_CASH_IN command using WFS_CMD_CIM_CASH_IN_END.

| Step | Customer | Application | XFS Command |
| --- | --- | --- | --- |
| 1. | Customer selects cash-in operation. |  | WFS_CMD_CIM_CASH_IN_START |
| 2. |  |  | * WFS_CMD_CIM_CASH_IN initiated with a long timeout (for example, WFS_INDEFINITE_WAIT) using WFSAsyncExecute The Service Provider implicitly opens the shutter. … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTOPEN) WFS_EXEE_CIM_INSERTITEMS event is sent when the shutter is fully open and the device is ready to begin accepting items. |
| 3. |  | Ask the customer to insert money. Application sets an insertion timer. |  |
| 4. | Customer does not insert money. |  |  |
| 5. |  | The insertion timer elapses | WFSCancelAsyncRequest is executed to end the WFS_CMD_CIM_CASH_IN command. |
| 6. |  |  | * If command cancellation is supported the WFS_CMD_CIM_CASH_IN completes with WFS_ERR_CANCELED. WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTCLOSED) |
| 7. |  | Transaction cancelled | WFS_CMD_CIM_CASH_IN_END |
| 8. |  | End of transaction. |  |

### First Bunch Not Taken

In this flow, the user does not take the first returned bunch within the required time, therefore the application cancels the WFS_CMD_CIM_CASH_IN command. The same sequence can be extended to any bunch other than the last bunch as this would complete the WFS_CMD_CIM_CASH_IN command; each time a new bunch is presented a new presentation timer should be set.

| Step | Customer | Application | XFS Commands and Events |
| --- | --- | --- | --- |
| 1.-3. | See No Items Inserted |  |  |
| 4. | Customer inserts money |  | If bItemsInsertedSensor == TRUE: WFS_SRVE_CIM_ITEMSINSERTED The Service Provider implicitly closes the shutter. … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTCLOSED) The bill recognition begins. |
| 5. |  | Insertion timer cancelled |  |
| 6. |  |  | As a result of the bill processing n bunches of items must be returned to the customer. |
| 7. |  |  | WFS_EXEE_CIM_INPUTREFUSE |
| 8. |  |  | Return bunch 1 of items to customer. The Service Provider implicitly opens the shutter and implicitly presents the bunch of items. … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTOPEN) WFS_SRVE_CIM_ITEMSPRESENTED |
| 9. |  | Tell the customer that the items were not accepted, and to take the items. The customer should be informed that the items will be returned in multiple bunches. If there are additional bunches to deliver then this can be determined from the output parameter of the WFS_SRVE_CIM_ITEMSPRESENTED event. Presentation timer set |  |
| 10. | Customer does not take the items | The presentation timer elapses | WFSCancelAsyncRequest is executed to end the WFS_CMD_CIM_CASH_IN command. |
|  |  |  | * If command cancellation is supported the WFS_CMD_CIM_CASH_IN completes with WFS_ERR_CANCELED. WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTCLOSED) |
| 11. |  | All items are retracted. | WFS_CMD_CIM_RETRACT |
| 12. |  | End of transaction. |  |

### Last Bunch Taken

In this flow, two bunches are to be returned & the user takes all of the returned bunches within the required time, therefore WFS_CMD_CIM_CASH_IN command completes normally.

| Step | Customer | Application | XFS Commands and Events |
| --- | --- | --- | --- |
| 1.-9. | See First Bunch Not Taken |  |  |
| 10. | Customer takes the bunch |  | WFS_SRVE_CIM_ITEMSTAKEN The Service Provider implicitly closes the shutter. … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTCLOSED) |
| 11. |  | Presentation timer cancelled | Return bunch 2 of items to customer. The Service Provider implicitly opens the shutter and implicitly presents the bunch of items. … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTOPEN) WFS_SRVE_CIM_ITEMSPRESENTED |
| 12. |  |  | * WFS_CMD_CIM_CASH_IN completes with WFS_SUCCESS. |
| 13. | Customer takes the bunch of items. |  |  |
| 14. |  |  | WFS_SRVE_CIM_ITEMSTAKEN The Service Provider implicitly closes the shutter. … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTCLOSED) |
| 15. |  | Display the amount recognized so far. |  |
| 16. |  | Ask the customer for further actions: If the customer wants to deposit the amount: Continue with step 17. If the customer wants to get back all items inserted so far see table "Cancellation by Customer (Implicit Shutter Control)" |  |
| 17. |  | Transport the money into the cash units of type WFS_CIM_TYPERECYCLING / WFS_CIM_TYPECASHIN. | WFS_CMD_CIM_CASH_IN_END |
| 18. |  | Credit the money to the customer's account. |  |
| 19. |  | End of transaction. |  |

### Clarifications for Mixed Media Cancellation by Customer

The following table describes the flow of a Mixed Media transaction where the customer wants all the items to be returned. In this case the returned items must be explicitly presented by the application.

This flow covers the following cases:

- bItemsInsertedSensor == TRUE, bItemsTakenSensor == TRUE

- bCompound == TRUE, wMixedMode == WFS_CIM_IPMMIXEDMEDIA

- WFSCIMPOSCAPS.bPresentControl == FALSE

| Step | Customer/ Application | CIM Commands and Events | IPM Commands and Events |
| --- | --- | --- | --- |
| 1.-12. | As per Mixed Media OK Transaction. |  |  |

### Clarifications for Mixed Media Cancellation by Customer on Cash Part Only

The following table describes the flow of a Mixed Media transaction where the customer wants the cash items to be returned but deposit the check items. In this case the returned items are implicitly presented by the Service Provider.

This flow covers the following cases:

- bItemsInsertedSensor == TRUE, bItemsTakenSensor == TRUE

- wMixedMode == WFS_CIM_IPMMIXEDMEDIA

- WFSCIMPOSCAPS.bPresentControl == TRUE

| Step | Customer/ Application | CIM Commands and Events | IPM Commands and Events |
| --- | --- | --- | --- |
| 1.-12. | As per Mixed Media OK transaction |  |  |

### Clarifications for Mixed Media Multiple Refused Items

The following table describes the flow of a Mixed Media transaction where items are rejected during the transaction. Additionally, the number of items refused may be greater than the number of items that can be presented at the output position. In this case the returned items must be explicitly presented by the application.

This flow covers the following cases:

- bShutterControl == TRUE, bItemsInsertedSensor == TRUE, bItemsTakenSensor == TRUE

- bCompound == TRUE, wMixedMode == WFS_CIM_IPMMIXEDMEDIA

- WFSCIMPOSCAPS.bPresentControl == FALSE

| Step | Application/ Customer | CIM Commands and Events | IPM Commands and Events |
| --- | --- | --- | --- |
| 1. | Customer selects Mixed Media transaction. | WFS_CMD_CIM_CASH_IN_START |  |
| 2. |  | * WFS_CMD_CIM_CASH_IN initiated (The shutter is not opened until WFS_CMD_IPM_MEDIA_IN is called.) … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTOPEN) | * WFS_CMD_IPM_MEDIA_IN initiated Service Provider opens the input shutter. … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTOPEN) |
| 3. |  | WFS_EXEE_CIM_INSERTITEMS event is sent when the shutter is fully open and the device is ready to begin accepting items. | WFS_EXEE_IPM_NOMEDIA This event specifies that media must be inserted into the device in order for the execute command to proceed. |
| 4. | Ask the customer to insert items. |  |  |
| 5. | Customer inserts items. |  |  |
| 6. |  | WFS_SRVE_CIM_ITEMSINSERTED | WFS_EXEE_IPM_MEDIAINSERTED |
| 7. |  | The Service Provider closes the input shutter and the device begins processing the inserted items. … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTCLOSED) | … WFS_SRVE_IPM_SHUTTERSTATUSCHANGED(WFS_IPM_SHTCLOSED) Send one WFS_EXEE_IPM_MEDIADATA event for every check item identified. |
| 8. | Items are refused. | WFS_EXEE_CIM_INPUTREFUSE event sent with appropriate lpusReason parameter. Items that are not bills or checks are rejected with WFS_CIM_INVALIDBILL. | WFS_EXEE_IPM_MEDIAREFUSED |
| 9. |  | * WFS_CMD_CIM_CASH_IN completes. | * WFS_CMD_IPM_MEDIA_IN completes. |
| 10. | Application chooses to return refused items now. | * WFS_CMD_CIM_PRESENT_MEDIA initiated (No physical action may take place until the WFS_CMD_IPM_PRESENT_MEDIA command.) | * WFS_CMD_IPM_PRESENT_MEDIA initiated |
| 11. | Each bunch of items presented. | … WFS_SRVE_CIM_SHUTTERSTATUSCHANGED(WFS_CIM_SHTOPEN) WFS_SRVE_CIM_ITEMSPRESENTED | … WFS_SRVE_IPM_SHUTTERSTATUSCHANGED(WFS_IPM_SHTOPEN WFS_EXEE_IPM_MEDIAPRESENTED |
| 12. | All but last bunch of items taken. | WFS_SRVE_CIM_ITEMSTAKEN | WFS_SRVE_IPM_MEDIATAKEN |
| 13. |  | * WFS_CMD_CIM_PRESENT_MEDIA completes. | * WFS_CMD_IPM_PRESENT_MEDIA completes. |
| 14. | Last bunch of items taken. | WFS_SRVE_CIM_ITEMSTAKEN | WFS_SRVE_IPM_MEDIATAKEN |
| 15. | Transaction continues from step 123. in the Mixed Media OK transaction. |  |  |

## Card Dispenser

Class Name		CRD

## Bar Code Reader

Class Name		BCR

### Clarifications for C - Header File

..

/* Values of WFSBCRSTATUS.dwGuidLights [...]

WFSBCRCAPS.dwGuidLights [...],

WFSBCRSETGUIDLIGHT.wGuidLight */

#define     WFS_BCR_GUIDANCE_NOT_AVAILABLE      (0x00000000)

#define     WFS_BCR_GUIDANCE_OFF                (0x00000001)

/* The following value (WFS_BCR_GUIDANCE_ON) should NOT be used */

#define     WFS_BCR_GUIDANCE_ON                 (0x00000002)

#define     WFS_BCR_GUIDANCE_SLOW_FLASH         (0x00000004)

#define     WFS_BCR_GUIDANCE_MEDIUM_FLASH       (0x00000008)

#define     WFS_BCR_GUIDANCE_QUICK_FLASH        (0x00000010)

#define     WFS_BCR_GUIDANCE_CONTINUOUS         (0x00000080)

#define     WFS_BCR_GUIDANCE_RED                (0x00000100)

#define     WFS_BCR_GUIDANCE_GREEN              (0x00000200)

#define     WFS_BCR_GUIDANCE_YELLOW             (0x00000400)

#define     WFS_BCR_GUIDANCE_BLUE               (0x00000800)

#define     WFS_BCR_GUIDANCE_CYAN               (0x00001000)

#define     WFS_BCR_GUIDANCE_MAGENTA            (0x00002000)

#define     WFS_BCR_GUIDANCE_WHITE              (0x00004000)

#define     WFS_BCR_GUIDANCE_ENTRY              (0x00100000)

#define     WFS_BCR_GUIDANCE_EXIT               (0x00200000)

## Item Processing Module

Class Name		IPM

The IPM device class uses the concept of a Media-In transaction to track and control a customer’s interaction with the device. A Media-In transaction consists of one or more WFS_CMD_IPM_MEDIA_IN commands. The transaction is initiated by the first WFS_CMD_IPM_MEDIA_IN command and remains active until the transaction is either confirmed through WFS_CMD_IPM_MEDIA_IN_END, or terminated by WFS_CMD_IPM_MEDIA_IN_ROLLBACK, WFS_CMD_IPM_RETRACT_MEDIA or WFS_CMD_IPM_RESET. While a transaction is active the WFS_INF_IPM_TRANSACTION_STATUS command reports the status of the current transaction. When a transaction is not active the WFS_INF_IPM_TRANSACTION_STATUS command reports the status of the last transaction as well as some current status values.

There are primarily two types of devices supported by the IPM, those devices with a stacker and those without.

In this specification the terms “long edge” and “short edge” are used to describe the orientation of a check and the length of its edges. The diagram below illustrates these definitions.

### Clarifications for WFS_INF_IPM_STATUS

lppPositions
…

wTransport
Supplies the state of the transport mechanism as one of the following values. The transport is defined as any area leading to or from the position:

### Clarifications for WFS_INF_IPM_TRANSACTION_STATUS

Description	This command is used to request the status of the current or last media-in transaction as well as current status values outside a transaction. A media-in transaction consists of one or more WFS_CMD_IPM_MEDIA_IN commands. A media-in transaction is initiated by the WFS_CMD_IPM_MEDIA_IN command and remains active until the transaction is either confirmed through the WFS_CMD_IPM_MEDIA_IN_END command, or cancelled by the WFS_CMD_IPM_MEDIA_IN_ROLLBACK, the WFS_CMD_IPM_RETRACT_MEDIA or the WFS_CMD_IPM_RESET command. Multiple calls to the WFS_CMD_IPM_MEDIA_IN command can be made while a transaction is active to obtain additional items from the customer.

The following values returned by this command can change after the media-in transaction has ended if items are later moved in the device:

WFSIPMTRANSSTATUS.usMediaOnStacker
WFSIPMTRANSSTATUS.lpszExtra
WFSIPMMEDIASTATUS.wMediaLocation
WFSIPMMEDIASTATUS.usBinNumber
WFSIPMMEDIASTATUS.wCustomerAccess

Output Param	…

usMediaOnStacker
Contains the total number of media items currently on the stacker (including usLastMediaAddedToStacker), or WFS_IPM_MEDIANUMBERUNKNOWN if it is unknown. This count only applies to devices with stackers and is persistent. This value can change outside of a media-in transaction as the media moves within the device.

lppMediaInfo
Pointer to a NULL-terminated array of pointers to WFSIPMMEDIASTATUS structures. This array contains details of the media items processed during the current or last transaction (depending on the value of wMediaInTransaction). The array contains one element for every item that has been allocated a media ID (i.e. IPM items that have been reported to the application). If there are no media items then lppMediaInfo is NULL. The WFSIPMIMAGEDATA structure is described in the WFS_CMD_IPM_READ_IMAGE command section. The media info is available until a new transaction is started with the WFS_CMD_IPM_MEDIA_IN command. The media location information may be updated after a transaction is completed, e.g. if media that was presented to the customer is subsequently retracted. The media info is persistent.

typedef struct _wfs_ipm_mediastatus
	{
	USHORT			usMediaID;
	WORD				wMediaLocation;
	USHORT			usBinNumber;
	ULONG				ulCodelineDataLength;
	LPBYTE			lpbCodelineData;
	WORD				wMagneticReadIndicator;
	LPWFSIPMIMAGEDATA	*lppImage;
	WORD				fwInsertOrientation;
	LPWFSIPMMEDIASIZE	lpMediaSize;
	WORD				wMediaValidity;
	WORD				wCustomerAccess;
	} WFSIPMMEDIASTATUS, *LPWFSIPMMEDIASTATUS;

wMediaLocation
Specifies the location of the media item. This value can change outside of a media-in transaction as the media moves within the device. This value is specified as one of the following values:

Value	Meaning

...

usBinNumber
If wMediaLocation is WFS_IPM_LOCATION_BIN then this field contains the bin number where the media was stored. This value can change outside of a media-in transaction as the media moves within the device.

lpMediaSize
Pointer to a WFSIPMMEDIASIZE structure that specifies the size of the media item. lpMediaSize is NULL if the device does not support media size measurement.

typedef struct _wfs_ipm_media_size
	{
	ULONG				ulSizeX;
	ULONG				ulSizeY;
	} WFSIPMMEDIASIZE, *LPWFSIPMMEDIASIZE;

ulSizeX
Specifies the width length of the long edge of the media in millimeters, or zero if unknown.

ulSizeY
Specifies the height length of the short edge of the media in millimeters, or zero if unknown.

wCustomerAccess
Specifies if the media item has been in customer access since it was first deposited, e.g. it has been retracted from a position with customer access. This value can change outside of a media-in transaction as the media moves within the device. This value is specified as one of the following values:

Value	Meaning

….

lpszExtra
Pointer to a list of vendor-specific, or any other extended, information. The information is returned as a series of “key=value” strings so that it is easily extensible by Service Providers. Each string is null-terminated, with the final string terminating with two null characters. An empty list may be indicated by either a NULL pointer or a pointer to two consecutive null characters. This value can change outside of a media-in transaction as the media moves within the device.

### Clarifications for WFS_INF_IPM_MEDIA_IN

Error Codes	In addition to the generic error codes defined in [Ref. 1], the following error codes can be generated by this command:

Value	Meaning

WFS_ERR_IPM_ALLBINSFULL	All media bins are full unusable due to being full, missing or inoperative, so no further items can be accepted.

### Clarifications for WFS_CMD_IPM_READ_IMAGE

Output Param	…

lpMediaSize
Pointer to a WFSIPMMEDIASIZE structure that specifies the size of the media item. lpMediaSize is NULL if the device does not support media size measurement.

typedef struct _wfs_ipm_media_size
	{
	ULONG				ulSizeX;
	ULONG				ulSizeY;
	} WFSIPMMEDIASIZE, *LPWFSIPMMEDIASIZE;

ulSizeX
Specifies the width length of the long edge of the media in millimeters, or zero if unknown.

ulSizeY
Specifies the height length of the short edge of the media in millimeters, or zero if unknown.

### Clarifications for WFS_SRVE_IPM_MEDIATAKEN

Comments	Note that since this event can occurs after the completion of a function that includes a media eject, it is not an execute event.

### Clarifications for WFS_EXEE_IPM_MEDIAREFUSED

Event Param	LPWFSIPMMEDIAREFUSED lpMediaRefused;

typedef struct _wfs_ipm_media_refused
	{
	WORD				wReason;
	WORD				wMediaLocation;
	BOOL				bPresentRequired;
	LPWFSIPMMEDIASIZE	lpMediaSize;
	} WFSIPMMEDIAREFUSED, *LPWFSIPMMEDIAREFUSED;

wReason
Specified as one of the following values:

Value	Meaning

…

WFS_IPM_REFUSED_TOOLONG	The media item (or bunch of items) long edge was too long.

WFS_IPM_REFUSED_TOOSHORT	The media item (or bunch of items) long edge was too short.

WFS_IPM_REFUSED_TOOWIDE	The media item (or bunch of items) short edge was too wide.

WFS_IPM_REFUSED_TOONARROW	The media item (or bunch of items) short edge was too narrow.

lpMediaSize
Pointer to a WFSIPMMEDIASIZE structure that specifies the size of the refused media (or bunch of media). lpMediaSize is NULL if the device does not support media size measurement.

typedef struct _wfs_ipm_media_size
	{
	ULONG				ulSizeX;
	ULONG				ulSizeY;
	} WFSIPMMEDIASIZE, *LPWFSIPMMEDIASIZE;

ulSizeX
Specifies the width length of the long edge of the media in millimeters, or zero if unknown.

ulSizeY
Specifies the height length of the short edge of the media in millimeters, or zero if unknown.

### Clarifications for WFS_EXEE_IPM_MEDIADATA

Event Param	…

lpMediaSize
Pointer to a WFSIPMMEDIASIZE structure that specifies the size of the media item. lpMediaSize is NULL if the device does not support media size measurement.

typedef struct _wfs_ipm_media_size
	{
	ULONG				ulSizeX;
	ULONG				ulSizeY;
	} WFSIPMMEDIASIZE, *LPWFSIPMMEDIASIZE;

ulSizeX
Specifies the width length of the long edge of the media in millimeters, or zero if unknown.

ulSizeY
Specifies the height length of the short edge of the media in millimeters, or zero if unknown.