CEN

CWA 15748

WORKSHOP	November 2007

AGREEMENT

ICS 35.240.50

English version

Extensions for Financial Services (XFS) interface specification

Release 3.10

Clarifications for MIB Device Specific Definitions

Revision 3.10.01

October 2011

This CEN Workshop Agreement has been drafted and approved by a Workshop of representatives of interested parties, the constitution of which is indicated in the foreword of this Workshop Agreement.

The formal process followed by the Workshop in the development of this Workshop Agreement has been endorsed by the National Members of CEN but neither the National Members of CEN nor the CEN Management Centre can be held accountable for the technical content of this CEN Workshop Agreement or possible conflicts with standards or legislation.

This CEN Workshop Agreement can in no way be held as being an official standard developed by CEN and its Members.

This CEN Workshop Agreement is publicly available as a reference document from the CEN Members National Standard Bodies.

CEN members are the national standards bodies of Austria, Belgium, Cyprus, Czech Republic, Denmark, Estonia, Finland, France, Germany, Greece, Hungary, Iceland, Ireland, Italy, Latvia, Lithuania, Luxembourg, Malta, Netherlands, Norway, Poland, Portugal, Slovakia, Slovenia, Spain, Sweden, Switzerland and United Kingdom.

EUROPEAN COMMITTEE FOR STANDARDIZATION

COMITÉ EUROPÉEN DE NORMALISATION

EUROPÄISCHES KOMITEE FÜR NORMUNG

Management Centre: rue de Stassart, 36 B-1050 Brussels

© 2008 CEN All rights of exploitation in any form and by any means reserved worldwide for CEN national Members.

Ref. No. CWA 15748-1:2007 D/E/F

Table of Contents

Introduction	3

Generic Clarifications	4

Architectural and Implementation Issues	5

XFS MIB Architecture and SNMP Extensions – Programmer’s Reference	6

XFS MIB Device Specific Definitions	7

Cash Dispensers	7

Clarifications for CDM Detailed Device Status Change Trap	7

Personal Identification Number Keypads (PIN Pads)	8

Clarifications for xfsPINStatusTable: States	8

Cash In Module	9

Clarifications for Appendix A – CIM MIB sub-tree	9

Item Processing Module	10

Clarifications for xfsIPMStatusTable: States	10

Clarifications for IPM Detailed Device Status Change Trap	10

Clarifications for IPM Reset Device Complete Trap	10

# Introduction

These release notes provide clarifications and explanations for the XFS MIB parts of the CWA 15748. When a XFS MIB part is updated for functional changes then all clarifications contained in this clarification document at the current revision level for that XFS MIB part will be incorporated into the new revision.

The clarification will be incorporated into the appropriate section and be printed as bold and underlined.  In any case where the clarification affects the MIB itself, the updated MIB has been embedded in this document.

# Generic Clarifications

No clarifications available.

# Architectural and Implementation Issues

No clarifications available.

# XFS MIB Architecture and SNMP Extensions – Programmer’s Reference

No clarifications available.

# XFS MIB Device Specific Definitions

## Cash Dispensers

Class Name		CDM

### Clarifications for CDM Detailed Device Status Change Trap

CDM Detailed Device Status Change Trap: an example

…

| xfsMIBRoot.3.1.3.5 | (xfsMIBRoot.xfsTrap.xfsTrapV1.xfsCommonTrapVars.xfsCommonTrapManagedServiceType) |
| --- | --- |
|  | 31 (WFS_CDM_SELFSERVICEBILL) |

## Personal Identification Number Keypads (PIN Pads)

Class Name		PIN

### Clarifications for xfsPINStatusTable: States

…

xfsPINStatusDevicePosition (8)

It contains the device position. It is a numeric type field. Allowed values are:

Value				Meaning

xfsPINDevicePosNotSupported (4)      	The physical device does not have the capability of detecting the  position.

## Cash In Module

Class Name		CIM

### Clarifications for Appendix A – CIM MIB sub-tree

…

-- ********************************************************************

--   CIM Status #defines

-- ********************************************************************

…

IxfsCIMIntermediateStackerStatus ::= INTEGER

{

xfsCIMIsEmpty(1),

xfsCIMIsNotEmpty(2),

xfsCIMIsFull(3),

xfsCIMIsUnknown(45),

xfsCIMIsNotSupported(56)

}

…

-- ***********************************************************************

-- CIM Device Status Table

-- ***********************************************************************

…

-- 1.3.6.1.4.1.16213.2.13.1.2.1.6

xfsCIMStatusIntermediateStacker OBJECT-TYPE

SYNTAX IxfsCIMIntermediateStackerStatus

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Intermediate Stacker Status.

xfsCIMISEmpty(1),

xfsCIMISNotEmpty(2),

xfsCIMISFull(3),

xfsCIMISUnknown(45),

xfsCIMISNotSupported(56)."

::= { xfsCIMStatusEntry 6 }

## Item Processing Module

Class Name		IPM

### Clarifications for xfsIPMStatusTable: States

…

xfsIPMStatusRebBuncher (12)

It contains the state of the rebuncher (return stacker). The rebuncher is where media items are re-bunched ready for return to the customer. Allowed values are:

Value										Meaning

…

### Clarifications for IPM Detailed Device Status Change Trap

…

xfsMIBRoot.xfsManagedService.xfsIPM.xfsIPMV1.xfsIPMStatusTable.xfsIPMStatusEntry.xfsIPMStatusReBbuncher.xfsIPMStatusManagedServiceName (23)

It contains the state of the rebuncher (return stacker). The rebuncher is where media items are re-bunched ready for return to the customer. It is a numeric type field.

…

| xfsMIBRoot.2.16.1.2.1.12.Index | (xfsMIBRoot.xfsManagedService.xfsIPM.xfsIPMV1.xfsIPMStatusTable.xfsIPMStatusEntry.xfsIPMStatusReBbuncher.xfsIPMStatusManagedServiceName) |
| --- | --- |
|  | 1 (xfsIPMReBbuncherEmpty) |

…

### Clarifications for IPM Reset Device Complete Trap

…

| xfsMIBRoot.2.16.1.2.1.12.Index | (xfsMIBRoot.xfsManagedService.xfsIPM.xfsIPMV1.xfsIPMStatusTable.xfsIPMStatusEntry.xfsIPMStatusReBbuncher.xfsIPMStatusManagedServiceName) |
| --- | --- |
|  | 1 (xfsIPMReBbuncherEmpty) |

…